7420.3 REV-2 CHG-7

APPENDIX 34

___________________________________________________________________________

Section 8 Moderate Rehabilitation Program Eligible

and Ineligible Work Items and Costs

1. Background.  For the purposes of the Section 8 Moderate Rehabilitation

Program, "moderate rehabilitation" is rehabilitation involving the

expenditure of at least $1,000 per housing unit to either:  (1)

upgrade substandard housing to a decent, safe and sanitary condition

to comply with the Housing Quality Standards (HQS) or other standards

(such as local codes) specified in the PHA's application and approved

by HUD; or (2) repair or replace major building systems or components

in danger of failure.  This Appendix clarifies the rehabilitation work

items and costs which are eligible under the Moderate Rehabilitation

Program for amortization through the Contract Rents.

2. Work Items Eligible for Inclusion in the Minimum Expenditure of $1,000

Per Unit.  In order to be eligible for the Moderate Rehabilitation

Program, a unit must require a rehabilitation expenditure of at least

$1,000 for improvements, other than routine maintenance or luxury

items, necessary to bring the unit in compliance with the HQS or local

standards approved by HUD.  This $1,000 minimum expenditure per unit

includes the cost of materials and labor to perform the necessary and

eligible work in accordance with paragraphs 3 and 4, prorated costs of

improvements to common systems or areas affecting the unit such as

central heating systems and roofs, and the cost of caulking and

weatherstripping.  When prorating costs of work to be done to common

systems or areas, the total number of units in the structure,

including unassisted units, must be used to determine the cost

attributable to each assisted unit.  (For example, if a ten-unit

building requires common improvements costing $10,000 and only five

units are to be assisted, the amount attributable to each assisted

unit is $1,000.)

There are two exceptions to the requirement that a unit be substandard

in order to qualify for the Moderate Rehabilitation Program:

a. Standard units which require repair or replacement of major

building systems or components in danger of failing within two

years are eligible.  Work qualifying as a major building system

or component is limited to:

(1) The complete electrical rewiring of a unit;

___________________________________________________________________________

Page 112/81

7420.3 REV-2 CHG-7

APPENDIX 34

___________________________________________________________________________

(2) The installation of new plumbing supply or waste pipes in a

unit;

(3) The installation of a new heating distribution system,

including piping and ductwork, or the installation of a new

boiler or furnace;

(4) The installation of a new roof; or

(5)Replacement or major repair of exterior structural elements

which are essential to achieve a stable general condition

with no threat of further deterioration (for example,

removal and replacement of rotted materials, repointing of a

large area of exterior walls to eliminate water seepage,

major repair of unstable or deteriorated foundation walls,

or painting wood which has never been painted, stained or

treated).

b.For up to five percent of the units in a PHA's Moderate

Rehabilitation Program (total number of units in the Annual

Contributions Contract), costs to make units physically

accessible for handicapped or disabled tenants may be counted

toward the $1,1000 per unit minimum rehabilitation expenditure.

The rehabilitation must make the unit, including access to and

egress from that unit, barrier-free with respect to the handicap

or disability of the current tenant or individual expected to

occupy the unit.

In both exceptions specified above, the $1,000 minimum rehabilitation

expenditure per unit is still applicable and must not include routine

maintenance or luxury items, but may include prorated costs of common

improvements.

3.Work Items and Costs Eligible for Inclusion After the $1,000 per Unit

Minimum Expenditure Has Been Met.  Once the PHA has determined that a

unit requires at least $1,000 of rehabilitation in accordance with

paragraph 2 of this Appendix, an analysis must be made to determine

whether other necessary work items are feasible and whether the unit

can be assisted under the Moderate Rehabilitation Program.  The

following expense items are eligible for amortization through the

Contract Rents, but may not be counted toward the minimum per unit

expenditure of $1,000:

a.Exterior painting; interior painting directly related to an

eligible rehabilitation work item and not prohibited by paragraph

4; and air conditioning systems or equipment provided the PHA has

determined that air conditioning is typically provided in

unassisted, modestly designed housing for lower income families

in the locality.

___________________________________________________________________________

12/81                              Page 2

7420.3 REV-2 CHG-7

APPENDIX 34

___________________________________________________________________________

b.Rehabilitation financing costs and fees which are reasonable,

directly connected to financing the project, and required by the

lender.  In order to be considered reasonable, these costs and

fees must be less than 2 percent of the rehabilitation loan or

new mortgage amount.  (Escrows for taxes or other operating

expenses are excluded.)

c.Necessary rehabilitation related costs and fees which are

reasonable, directly connected with the eligible work items

approved by the PHA, and do not include costs for services the

PHA is required to provide owners or services available locally

at no expense.  Eligible expenses could include the cost of

building permits required by the local government and expenses

for necessary architectural and engineering services not provided

by the PHA.  Preparation of drawings by an architect is not

required in the Moderate Rehabilitation Program and would usually

be inappropriate considering the scope of rehabilitation to be

accomplished.  However, an architectural fee could be an eligible

expense if a substantial amount of rehabilitation will be

accomplished and drawings are necessary to accomplish the

eligible rehabilitation work items.

d.Energy conserving improvements which are practicable,

cost-effective and financially feasible.  The PHA must review

each owner proposal to determine compliance with the attached

Cost-Effective Energy Conservation Standards (Attachment 1).  If

the owner's proposal is found feasible at a gross rent lower than

the applicable Fair Market Rent, the PHA must analyze whether

additional energy conserving improvements are appropriate.

Although PHAs are encouraged to include energy conservation items

(including solar energy systems which are cost-effective and

economically feasible), these improvements generally will be

required only in connection with other rehabilitation work items

(e.g., installation of insulation in an exterior wall if that

wall will be made accessible or become exposed during the

rehabilitation).

e.Actual temporary relocation costs in accordance with 24 CFR

Section 882.407.  (PHA administrative expenses associated with

temporary relocation are reimbursed through administrative fees

and are not eligible for amortization through the Contract

Rents.)

f.Removal of architectural barriers to make a unit physically

accessible to a handicapped or disabled tenant or prospective

tenant.  Accessibility renovations may be included for any unit

which requires at least $1,000 to correct substandard conditions,

in addition to the five percent exception specified in paragraph

2(b) of this Appendix.

___________________________________________________________________________

Page 312/81

7420.3 REV-2 CHG-7

APPENDIX 34

___________________________________________________________________________

4.Ineligible Work Items and Costs.  Since PHAs will usually receive

proposals from owners of substandard units, such units will have

deficiencies ranging from deferred maintenance to structural problems.

In order to reimburse owners only for necessary improvements and thus

allow the most rehabilitation to be accomplished within the Fair

Market Rent constraints, the following work items are never eligible

for amortization through the Contract Rents:

a.Cleaning and janitorial services, or landscaping (such as

reseeding of lawns or planting of trees and shrubbery) not

directly related to an eligible rehabilitation work item.

(Landscaping would be eligible if associated with the correction

of a severe erosion or drainage problem which affects the

structural integrity of the unit.)

b.Repair, replacement or installation of laundry equipment,

furniture, nondwelling equipment, and other amenities (e.g., play

equipment, recreational items and swimming pools), or drapes,

shades and other items subject to normal wear and tear.

c.Interior painting not directly related to an eligible

rehabilitation work item or necessary to correct lead based paint

deficiencies.  (The cost of painting an entire interior room is

eligible if a substantial portion of the wall surface needs

painting as a result of other eligible rehabilitation

activities.)

d.Costs attributable to owner labor (i.e., direct work or

supervision of the work), whether or not the owner is a licensed

contractor.

It is noted that the routine maintenance items listed above may be

required by the PHA to bring units up to full compliance with the HQS

or local standards, and that repair or installation of ineligible

amenities may be voluntarily performed by the owner in connection with

the Moderate Rehabilitation Program.  However, such work cannot be

included in the rehabilitation costs amortized through the Contract

Rents.  If routine maintenance items are required by the PHA, the

owner must agree to complete the work at the owner's expense.

5.PHA Determination of Eligible Work Items in Compliance with the HQS.

In order to maximize the amount of upgrading possible within the

financial constraints of the Moderate Rehabilitation Program, it is

essential that PHAs carefully develop standards for determining

eligible work items, especially if the HQS are being used.  For

example, the HQS require that the stove and refrigerator be "in proper

operating condition" and that this equipment be free of hazardous

conditions that endanger the tenants, such as gas or water leakage

___________________________________________________________________________

12/81                              Page 4

7420.3 REV-2 CHG-7

APPENDIX 34

___________________________________________________________________________

that could result in fires or electrical shock.  The HQS do not

specify standards for the age or appearance of appliances; stoves and

refrigerators should not be replaced if they currently work and are

adequate for cooking and refrigeration purposes.  It is noted that a

reserve for replacement can be included when calculating the Contract

Rent under the cost approach and this reserve should be sufficient to

replace or repair unit equipment during the 15-year contract term.

Another area requiring PHA judgment will be remodeling of kitchens and

bathrooms and installation of air conditioning equipment or carpeting.

Although remodeling will most likely increase the value of the

property and improve its desirability, PHAs should be satisfied that

remodeling is necessary to improve functional use (for example, by

providing adequate kitchen counter and storage space) and that the

resulting increase in Contract Rents is warranted.  Units requiring

only cosmetic kitchen or bathroom remodeling (or, likewise, only new

carpeting or air conditioning) should not be approved by PHAs, and

consideration should be given to the feasibility of leasing these

units under the Section 8 Existing Housing Program.

Owner applications which propose changing the present unit size

configuration (converting existing rooms into bedrooms or dividing an

existing room into two rooms to add a bedroom) must be reviewed

carefully.  Converting an existing room into a bedroom is an eligible

work item if the room being converted is not otherwise used for living

purposes.  For example, a basement or porch can be converted into a

bedroom; conversion of a dining room or den is ineligible and an

inappropriate way to qualify the unit for a higher rent.  Dividing an

existing room into two rooms to add a bedroom is likewise not

acceptable.  These alterations do not substantially improve the

family's living conditions by increasing the living space in the unit

and are not eligible work items for the Moderate Rehabilitation

Program.  In limited circumstances (e.g., units with especially large

rooms), the ineligible alterations listed above may be considered

appropriate.  In such cases, PHAs must request written HUD Field

Office approval to deviate from the stated policy.

Although the HQS do not specify minimum room sizes, the PHA will need

to determine that existing rooms are of adequate size and arrangement

for their intended function (i.e., the available space does not hinder

furniture placement and the normal use of space).  If it is desirable

to increase the unit's size by adding one or more bedrooms, the

rehabilitation must conform with the minimum room size standards

contained in HUD Handbook 4940.4, Minimum Design Standards (MDS) for

Rehabilitation of Residential Properties, or with other local

standards approved by the HUD Field Office.

PHAs are encouraged to use the MDS for additional guidance in

interpreting whether units currently meet the HQS or local standards,

___________________________________________________________________________

Page 512/81

7420.3 REV-2 CHG-7

APPENDIX 34

___________________________________________________________________________

the necessary extent of rehabilitation, and appropriate work

specifications.  In addition, Chapter 5 of the PHA Existing Housing

Administrative Practices Handbook 7420.7, provides clarification of

the HQS and PHA standards for determining unit compliance.  The HQS

tenant preference standards are applicable only to the Existing

Housing Program; the PHA will be responsible for making all

determinations of HQS acceptability for the Moderate Rehabilitation

Program.  Guidance in providing accessibility to the handicapped is

contained in "Specifications for Making Buildings and Facilities

Accessible to, and Useable by, the Physically Handicapped" (ANSI

A117.1).  This publication is available from the American National

Standards Institute, Inc., 1430 Broadway, New York, New York 10018.

___________________________________________________________________________

12/81                              Page 6

7420.3 REV-2 CHG-7

ATTACHMENT 1

___________________________________________________________________________

HOUSING AND URBAN

DEVELOPMENT

Office of the Secretary

24 CFR Part 39

Cost-Effective Energy Conservation

Standards

AGENCY:  Office of the Secretary, (HUD).

ACTION:  Interim rule.

__________________________________________________

SUMMARY:  The Secretary is prescribing

cost-effective energy conservation and

efficiency standards for certain HUD

programs and is inviting public comment

on these standards before they become

effective.  With the exception of one

program in which the Secretary has

determined that these standards are

needed as a matter of policy, adoption

and implementation of the standards are

required by recently enacted legislation.

COMMENT DUE:  June 11, 1979.

EFFECTIVE DATE:  July 9, 1979.

FOR FURTHER INFORMATION CONTACT:

Joseph Shermen, Director, Division of

Energy, Building Technology and

Standards Research, Office of Policy

Development and Research, (202)

755-6443 or Duane Keplinger, Acting

Director, Office of Architecture and

Engineering Standards, Office of

Housing, (202) 755-5718, Department of

Housing and Urban Development, 451

Seventh Street, S.W., Washington, D.C.

20410.

SUPPLEMENTARY INFORMATION:  The

Housing and Community Development

Amendments of 1978, 42 U.S.C. 1425(b)

et seq., Pub. L. 95-557 (1978

Amendments) require the Secretary to

___________________________________________________________________________

Page 1 (1 of 5)12/81

HUD-Wash., D.C.

7420.3 REV-2 CHG-7

ATTACHMENT 1

___________________________________________________________________________

prescribe cost-effective energy

conservation and efficiency standards

governing improvements and

rehabilitation in four HUD programs.  In

addition, the Secretary has determined

as a matter of policy that the standards

are needed in a fifth program to assure

conservation of energy resources.  Thus,

the standards, which are set forth as an

Appendix to this rule, will apply to (1)

rehabilitation loans under Section 312 of

the Housing Act of 1964 (see Section

101(a)(7) of the 1978 Amendments); (2)

rehabilitation loans under Section 203(k)

of the National Housing Act (see Section

101(c)(1) of the 1978 Amendments); (3)

Federal financial assistance for troubled

multifamily housing projects (see

Section 201(d)(6) of the 1978

Amendments); (4) Federal financial

assistance for substantial rehabilitation

projects under Section 8 of the United

States Housing Act of 1937 (see Section

206(d)(1) of the 1978 Amendments); and

(5) Federal financial assistance for

moderate rehabilitation protects under

Section 8 of the United States Housing

Act of 1937 (Secretary's policy

determination).

The Secretary has determined that the

standards are urgently needed and must

be adopted 60 days after publication

since the 1978 Amendments prohibit

Section 312 Rehabilitation loans after

July 27, 1979, covering work which does

not conform to the standards.  Hence, the

Section 312 program cannot continue

unless these standards have bean

adopted in sufficient time to acquaint

participants adequately with the cost-effective

energy provisions by that date.

Furthermore, the program under Section

203(k) of the National Housing Act

cannot be effectively implemented until

the standards have been adopted.  The

___________________________________________________________________________

Page 1 (2 of 5)12/81

HUD-Wash., D.C.

7420.3 REV-2 CHG-7

ATTACHMENT 1

___________________________________________________________________________

1978 Amendments also require

expedited implementation of the

Standards for the Section 8 Substantial

Rehabilitation Program and Troubled

Multifamily Housing Assistance

Program.

Before adopting the standards, the

Secretary is providing 30 days for

submittal of public comments.  If as a

result of comment, the Secretary

determines that change in the standards

is appropriate, their effectiveness will be

deferred by publication in the Federal

Register.  Otherwise, unless deferred by

such publication, the standards will

become effective without notification by

the Secretary 30 days after the close of

the period allowed for public comment,

that is 60 days after publication of this

document.  Interested parties are invited

to participate in this rulemaking

proceeding by submitting relevant

comments and suggestions to the Rules

Docket Clerk, Office of the General

Counsel, Department of Housing and

Urban Development, 431 Seventh Street,

S.W., Washington, D.C. 20410.  A

substantive response will be provided

as to each comment received through a

notice published in the Federal Register

or by mail.

The Secretary has determined that

this amendment does not affect the

quality of environment and a statement

to that effect has been prepared.  This

statement is available for inspection and

copying in the Office of the Rules

Docket Clerk at the above address.

The legislative review provisions of

Section 7(o) of the Department of HUD

Act, 42 U.S.C. 3535(o) have been met.

Accordingly Subtitle A is amended by

adding a new Part 39 to read as follows:

___________________________________________________________________________

Page 1 (3 of 5)12/81

HUD-Wash., D.C.

7420.3 REV-2 CHG-7

ATTACHMENT 1

___________________________________________________________________________

PART 39--COST-EFFECTIVE ENERGY

CONSERVATION AND

EFFECTIVENESS STANDARDS

Sec.

39.1  Title and purpose.

39.3  Authority.

39.5  Scope.

39.7  Standards.

Appendix I  Cost-Effective Energy Efficiency

(Conservation) Standards for

Rehabilitation of Residential Properties

39.1  Title and purpose.

The purpose of this Part is to set forth

cost-effective energy conservation and

efficiency standards applicable to HUD

programs.

39.3  Authority.

This Part implements the provisions of

the Housing and Community

Development Amendments of 1978, 42

U.S.C. 1425(b). et seq., Pub. L. 95-557.

39.5  Scope.

The standards apply to the following

programs:

(a) Rehabilitation loans under Section

312 of the Housing Act of 1964.

(b) Rehabilitation loans under Section

203(k) of the National Housing Act.

(c) Operating assistance for troubled

multifamily housing projects under

Section 201 of the Housing and

Community Development Amendments

of 1978.

(d) Financial assistance for

substantial rehabilitation projects under

Section 8 of the United States Housing

Act of 1937.

(e) Financial assistance for moderate

rehabilitation projects under Section 8 of

the United States Housing Act of 1937.

___________________________________________________________________________

Page 1 (4 of 5)12/81

HUD-Wash., D.C.

7420.3 REV-2 CHG-7

ATTACHMENT 1

___________________________________________________________________________

39.7  Standards.

The standards are set forth in detail in

the Appendix to this Part.

Appendix

Cost-Effective Energy Efficiency

(conservation) Standards for

Rehabilitation of Residential Properties

A-1.  General. (a) These standards

provide the mandatory levels of various

energy-efficiency features that are

considered as cost-effective in the

rehabilitation of residential properties.

The material is arranged to give

consideration to local climatic

conditions and fuel prices in

determining the measures to be used.

(b) It is realized that not all of the

requirements can be followed in all

situations due to existing building

conditions and funds available.

A-2.  Application. (a) It is the intent of

these standards to apply only to those

improvements proposed as part of the

___________________________________________________________________________

Page 1 (5 of 5)12/81

HUD-Wash., D.C.

7420.3 REV-2 CHG-7

ATTACHMENT 1

___________________________________________________________________________

rehabilitation, except those listed in A-3

below.  If new material or construction is

contemplated, conformance to these

standards is mandatory for these

features.

(b) It is not the intent to require

thermal improvement of construction

elements which would not ordinarily be

made accessible or become exposed

during the rehabilitation.

(c) The requirements contained herein

must be applied judiciously.  Any one or

more of the requirements may be

eliminated to the extent they are not

practical in rehabilitation situations,

considering economic feasibility,

program needs and the materials and

type of construction involved.  (Example:

The use of the following Tables for a

structure of 1500 square feet located in

an area of 4200 degree days with a fuel

cost of 4 cents per Kwh for electricity,

would require R-30 ceiling insulation for

certain programs.  If the structure

presently had no ceiling insulation, the

savings in fuel cost would be

approximately $1000 per year after

adding the R-30 insulation.  However, if

the present ceiling contained R-19

insulation, improvement to R-30 would

result in an annual reduction in fuel cost

of only $170.)  The reduced operating

cost must be weighed against the

amortization cost for the improvements

to determine cost-effectiveness for the

specific program terms.  (The energy

estimating methods contained in

Chapter 43 of the 1976 Systems

Handbook of the American Society of

Heating, Refrigerating and Air

Conditioning Engineers, Inc. were used

in preparing the above comparison.)

A-3.  Additional Criteria.  (a) In

addition to the measures shown in the

Tables, the following shall apply to

those structures which are not subjected

___________________________________________________________________________

12/81                         Page 2 ( 1 of 4)

HUD-Wash., D.C.

7420.3 REV-2 CHG-7

ATTACHMENT 1

___________________________________________________________________________

to the energy audit procedures described

in (b) below:  (1) Doors and windows

shall be weatherstripped to reduce

infiltration of air when weatherstripping

is inadequate or nonexistent.

(2) Caulk, gasket, or otherwise seal all

openings, cracks, or joints in the

building envelope when existing

materials are inadequate.  Replace all

loose or brittle caulking with new

elastomeric material.  Leave bottom edge

of siding uncaulked to allow moisture

drainage.

(3) Replacement heating, ventilating

and airconditioning (HVAC) system

supply and return pipes and ducts shall

be insulated whenever they run through

unconditioned spaces.  Pipe insulation

shall be R-2 or greater, and duct

insulation shall be R-4 or greater.  Duct

seams shall be tightly sealed to reduce

air leakage.

(4) Replacement heating systems,

burners and airconditioning systems

shall be of high-efficiency design, and

should be carefully sized to be no

greater than 15% oversize for the critical

design, bearing or cooling, except to

satisfy the manufacturers' next closest

nominal size.

(b) For mid-rise and high-rise

structures and for many low-rise

multifamily projects an energy audit

must be carried out to identify and

specify the energy and cost savings

which are estimated to result from

installing or accomplishing an energy

conservation measure.  All measures

determined to be cost-effective, given

the loan time and rate and in

accordance with paragraph A-2 shall be

accomplished.  Amortization cost of

improvements must be offset by reduced

operating costs attributable to the

improvements.  Generally, assistance in

conducting energy audits is available

through local supplies, energy officials

___________________________________________________________________________

12/81                         Page 2 ( 2 of 4)

HUD-Wash., D.C.

7420.3 REV-2 CHG-7

ATTACHMENT 1

___________________________________________________________________________

and utility companies.  The energy audit

shall cover all of the following energy

conservation measures, to the degree

feasible, except those that are not

appropriate for the building types or

equipment:

Installation of individual utility meters

or checkmeters

Ceiling insulation

Insulation of hot water end steam pipes

Insulation of heating and cooling ducts

in unheated spaces

Caulking and sealants in building joints

Weatherstripping doors and windows

Clock thermostats for units with

individual heating units

Storm windows or replacement of single

glazed windows with double glazed

windows

Flow restrictors for hot water lines to

shower heads or faucets

Thermostatic radiator valves

Replacement of incandescent fixtures in

public spaces with higher efficiency

lighting

Exterior insulation for water heaters

located in unheated spaces

Improved burners, controls, capacitors

and other improvements to

mechanical systems which will lower

fuel consumption or energy costs

A-4.  Use of Table 1.  In order to use

the recommendations shown in Table 3,

the Heating Index must be determined

from Table 2.  If the structure is

mechanically cooled, the Cooling Index

must also be selected from Table 2.  In

order to choose a Heating Index and a

Cooling Index from Table 2, it is

necessary to identify the appropriate

reference column number.  Table 1

presents those column reference

numbers.  At the top of Table 1 are unit

prices for oil, gas, and electricity

(electricity prices apply to resistance

electric heating and air conditioning

___________________________________________________________________________

12/81                         Page 2 ( 3 of 4)

HUD-Wash., D.C.

7420.3 REV-2 CHG-7

ATTACHMENT 1

___________________________________________________________________________

only) expressed in cents per gallon, per

therm, and per Kwh, respectively.  The

costs used in choosing the proper

reference column number should be the

marginal cost--the cost of the last unit

of fuel purchased in the heating or

cooling season--and should include all

taxes and fuel adjustment charges.

Choose reference column numbers for

the Heating Index by selecting the

reference number directly below the fuel

costs in the row representing the

financing terms to be used.  Similarly

select a Cooling Index reference column

by noting the number in the row of the

applicable financing terms that is

directly below the unit price of cooling

energy.  When fuel costs exceed those

shown in the Table, use the last column

on the right of the Table.

A-5.  Table 2--Selection of Heating

and Cooling Indices.  At the top of Table

2 are column reference numbers.  Along

the left side of Table 2 are Degree Days

for heating and Cooling Hours for

cooling.  The Heating Index is derived by

reading down the correct reference

column and across for the applicable

Degree Day entry.  The point of

convergence indicates the Heating

Index.  The reference column for cooling

and the Cooling Hours are used in the

same manner to determine the Cooling

Index.

A-6.  Table 3--Selection of

Recommended Measures.  The levels of

protection are determined by reading

horizontally across the Table from the

Heating Index shown on the left side of

the Table.  For ceiling insulation the

applicable Cooling Index column must be

used with the appropriate Heating

Index.  If mechanical cooling is not

contemplated, use the column for

Cooling Index 1.

BILLING CODE: 4210-01-04

___________________________________________________________________________

12/81                         Page 2 ( 4 of 4)

HUD-Wash., D.C.

7420.3 REV-2 CHG-7

ATTACHMENT 1

__________________________________________________________________________

Table 1

FUEL PRICES AND REFERENCE COLUMNS FOR HEATING AND COOLING INDICES

********************************************************************

* *

* *

* *

* *

* *

* *

* *

* *

* *

* *

* *

*       GRAPHICS  MATERIAL  IN  ORIGINAL  DOCUMENT  OMITTED        *

* *

* *

* *

* *

* *

* *

* *

* *

* *

* *

* *

********************************************************************

__________________________________________________________________________

Page 312/81

HUD-Wash., D.C.

7420.3 REV-2 CHG-7

ATTACHMENT 1

__________________________________________________________________________

Table 2

HEATING AND COOLING INDICES

********************************************************************

* *

* *

* *

* *

* *

* *

* *

* *

* *

* *

* *

*       GRAPHICS  MATERIAL  IN  ORIGINAL  DOCUMENT  OMITTED        *

* *

* *

* *

* *

* *

* *

* *

* *

* *

* *

* *

********************************************************************

__________________________________________________________________________

12/81                              Page 4

HUD-Wash., D.C.

7420.3 REV-2 CHG-7

ATTACHMENT 1

__________________________________________________________________________

Table 3

INSULATION:  R VALUES

********************************************************************

* *

* *

* *

* *

* *

* *

* *

* *

* *

* *

* *

*       GRAPHICS  MATERIAL  IN  ORIGINAL  DOCUMENT  OMITTED        *

* *

* *

* *

* *

* *

* *

* *

* *

* *

* *

* *

********************************************************************

__________________________________________________________________________

Page 512/81

HUD-Wash., D.C.

7420.3 REV-2 CHG-7

ATTACHMENT 1

__________________________________________________________________________

Notes to Table 3

(1) The addition of insulation to

ceilings which presently contain

insulation may not be cost-effective.  If

the additional insulation needed to

conform to the recommendations is less

than R-19, benefits are doubtful. (See

example in A-2(c).)

(2) For frame construction, the R value

of sheathing may be included in the

insulation R value of walls.

(3) Required only when wall covering,

interior or exterior, is removed and an

adequate vapor barrior is applied.

(When the existing wall contains any

amount of insulation, additional

insulation applied by blowing or pouring

is not recommended.)

(4) Extra insulation, up to R-19, is

recommended where practicable under

the conditions of Note (3) above.

(5) Three layers of glass not typically

cost-effective unless existing windows

are double glazed or complete

replacement of sash is contemplated.  If

replacing sash, should have insulating

frames.

(6) Storm doors or thermal doors

recommended if primary door is hollow-core

or is over 25 percent glass.

(7) Applicable only to individual living

unit doors opening directly to the

exterior.

(Section 7(d) of the Department of Housing

and Urban Development Act 42 U.S.C.

3535(d))

Issued at Washington, D.C. April 6, 1979.

Patricia Roberts Harris.

Secretary of Housing and Urban Development.

[Docket No. R-79-642]

[FR Doc.79-14582 Filed 5-9-79 8:45 am.]

__________________________________________________________________________

12/81                              Page 6

HUD-Wash., D.C.