CHAPTER 8.  STRUCTURAL SYSTEMS

PERFORMANCE OBJECTIVES

8-1.    GENERAL.  Structural systems of all buildings, accessory

structures and building elements shall be consistent with the

MPS and provide the following:

a.  Sufficient strength, stability and rigidity to safely

support all anticipated loads with code required safety

factors;

b.  Resistance to fire, corrosion, decay and other

destructive forces through adequate protection;

c.  Reasonable durability for the expected life of the

rehabilitated structure; and

d.  Compliance with relevant health, safety and fire codes.

8-1                    2/85

CHAPTER 8.  STRUCTURAL SYSTEMS

SECTION 1:  MANDATORY STANDARDS

8-2.    STRUCTURAL MEMBERS.  Structural elements shall be in sound

condition and free of all deficiencies that could lead to

failures, dislodging or collapse.  Structural elements and

spaces shall be free of the penetration of water, moisture,

infestation or causes leading to decay.  Walls, floors,

chimneys and stairs shall not have major sag or be out of

plumb.  Structural elements shall not have been weakened by

fire, earthquake, wind or flooding.

NOTE:  Major bulging of exterior walls, doors or windows not

fitting properly in their frames, or large cracks and major

water staining may indicate major structural problems.

Project rehabilitation should not be undertaken prior to the

correction of structural deficiencies.  When extensive

repair or replacement of primary structural elements is

necessary to make a building safe and structurally sound, the

PHA shall consider the physical and financial feasibility of

rehabilitation prior to any further planning.

8-3.    STRUCTURAL SPACES.  The entrance or accumulation of water,

moisture or heat that would be conducive to the decay,

deterioration or failure of building structure shall not be

permitted.  Structural spaces shall be ventilated by natural

or mechanical means to effectively minimize these conditions.

There shall be permanent access to structural spaces, such as

attics and crawl spaces, for periodic inspection and

maintenance.  This access is to be limited to authorized

individuals.

a.  Foundations, Basement and Crawl Spaces.  Foundations,

basements and crawl spaces shall not permit entrance of

water and excessive moisture.  Exterior openings in

foundations and walls at grade or below grade, including

pipe, vent and conduit openings, shall be protected

against the passage of rodents or infestation.  For

technical requirements, see

2/85                                  8-2

paragraph 2-5.  Foundations, basements and crawl spaces

shall be in sound structural condition, adequately plumb

to carry anticipated loads, and free of major holes,

cracks, bulging, spalling, decay or moisture penetration.

All masonry walls shall be free of efflorescence.

b.  Decay Protection.  All habitable, mechanical and

structural spaces located below grade shall be protected

from the causes of decay by ventilation; waterproofing as

required; and drainage of water away from the building.

8-3                    2/85

SECTION 2:  PROJECT SPECIFIC STANDARDS

8-2B.   RESERVED.

8-3B.   STRUCTURAL SPACES.

a.  Reserved.

b.  Reserved.

c.  Security.  In locations where security problems are

prevalent, windows and openings on basements and crawl

spaces may be protected from unauthorized entry with

vandal-resistant grilles or similar systems.  Security

systems shall not be permanently fixed if the window or

opening is a required means of emergency egress from the

building.  For technical requirements, see paragraph 9-7.

2/85                                8-4