U.S. Department of Housing and Urban Development

PUBLIC AND INDIAN HOUSING

Special Attention of: Public  Transmittal Handbook No.:7485.2 REV-1

Housing Agencies; Indian Housing

Authorities; Regional Administrators;

Directors, Offices of  Regional            Issued:  February 4, 1985

Public Housing; Field Office

Managers; Housing Management Division

Directors; Chiefs, Assisted Housing

Management Branches; Directors, Offices

of Indian Programs

1.   This Transmits

The Public Housing Modernization Standards Handbook 7485.2 REV-

l, dated 2/85.

2.   Explanation of Material Transmitted:

This Handbook provides design, construction and environmental

criteria for the rehabilitation of public housing, including

Indian housing, projects under the Comprehensive Improvement

Assistance Program (CIAP).  The standards establish a basis for

uniformly evaluating the physical condition and energy

efficiency of public housing projects to determine current

rehabilitation needs.

Appendix 3 contains the procedures for public housing agencies

(PHAs) to conduct the Physical Needs Assessment (Modernization)

Survey, as required by Paragraph 3-8 of the Public Housing

Comprehensive Improvement Assistance Program Handbook 7485.1

REV-2, and the Survey Instrument to be used for preparation and

submission of an application for funding under the CIAP.  This

survey will provide data required and include compliance

standards for modernization.  Appendix 4 contains a suggested

Project Cost Estimate Worksheet.

This Handbook incorporates policy clarifications which the

Department has found necessary after additional experience in

administering the CIAP.  The significant changes involve format

and restrictions.  These are as follows:

o  The standards are now formatted to clearly separate

items which are mandatory from those which are project

specific.  Each technical chapter is divided into two

sections: Section 1, Mandatory Standards; and Section

2, Project Specific Standards.

o  A list of prohibited items clarifying which items2,

may not be funded under any circumstances is, R-9-1,

provided in Appendix 1.

o  The Survey Instrument has been redesigned for easier

use and will be available separately as a Form HUD-

52827 (1/85).ppendix 1.

3.  Applicability of Issuance:

This Handbook applies to modernization approved under the CIAP in

Federal Fiscal Year (FFY) 1985 and thereafter.

4.  Cancellation:

Public Housing Modernization Standards Handbook 7485.2 REV,

dated 6/82

5.  Information Collection:

The reporting requirements contained in Appendix 3 of

this Handbook have been approved by the Office of

Management and Budget (OMB).  The OMB Number is 2577-

0047.

Assistant Secretary for Public and

Indian Housing

Page 2

Handbook 7485.2 REV-1

U.S. Department of Housing and Urban Development

Office of Public and Indian Housing

Program Participants

and Departmental

Staff

Reprint-January 1988                    Public Housing

Modernization

Standards

U.S. Department of Housing and Urban Development

Office of Public and Indian Housing

___________________________________________________________________________

SPECIAL ATTENTION OF: Regional             TRANSMITTAL

Administrator; Directors, Offices of       Handbook No:  7485.2 CHG-2

Public Housing; Field Office Managers,     Issued:  March 29, 1993

Public Housing Agencies;Indian Housing Authorities

Directors, Indian Programs

___________________________________________________________________________

1.  This Transmits Handbook 7485.2 CHG-2.

2.  Summary.  Chapter 5, paragraph 5-3(b) is changed to revise

the requirements for metal protector composition material to

include materials other than asbestos millboard with equal

or higher properties of fire resistance and heat absorbance.

3.  Filing Instructions:

Remove                               Insert

Handbook 7485.2                      Handbook 7485.2 CHG-2

Pages 5-1 thru 5-8                   Pages 5-1 thru 5-9

dated 2/85                           dated 3/93

General Deputy Assistant Secretary

for Public and Indian Housing

___________________________________________________________________________

W-3-1, W-2(H), R-1, R-3-1(PIH), R-3-2, R-6, R-6-1,

R-6-2, R-7, R-7-1, R-7-2, R-9, R-9-1, 138-2, 138-7

W-3-1    Directives Management Officers--Headquarters and Regions,

library, ACIR (Advisory Commission on Intergovermental

Relations)

W-2      HQ Office Directors, Special Assistants,

those reporting directly to  Assistant Secretaries

R-1      Regional Administrators, Deputy Regional Administrators

R-3-1    Bulk shipment to Regional Offices for selective

distribution by program area

R-3-2    Directors, Administrative Services Divisions and

Administrative and Management Services Divisions, and

Administrative staffs

R-6      Category A offices - Office Managers and Deputy Office

Managers

R-6-1    Category A offices - bulk

R-6-2    Category A offices - Division Directors

R-7      Category B offices - Office Managers and Deputy Office

Managers

R-7-1    Category B offices - bulk

R-7-2    Category B offices - Division Directors

R-9      Directors and Deputy Directors, Offices of Indian Programs

R-9-1    Offices of Indian Programs - bulk

U.S. Department of Housing and Urban Development

PUBLIC AND INDIAN HOUSING

Special Attention of:       Transmittal Handbook No.:  7485.2 REV-1

CHG-1

Issued: February 8, 1988

1.    This Transmits

changes to chapter 10 of the Public Housing Modernization

Standards Handbook 7485.2 REV-1, dated 2/88.

2.    Explanation of Material Transmitted:

This change updates the references in this Handbook regarding

the Lead-Based Paint Regulations 24 CFR Parts 35 and 968.9,

dated August 1, 1986.

3.    Applicability of Issuance:

Change is applicable to all construction contracts in

comprehensive modernization programs which are executed on or

after the effective date of this change.

4.    Filing Instructions:

Remove:                         Insert:

Handbook 7485.2 REV-1           Handbook 7485.2 REV-l CHG-1

Page 10-3, dated 2/85            Page 10-3 dated 2/88

Page 10-4, dated 2/85            Page 10-4 dated 2/85

General Deputy Assistant Secretary

for Public and Indian Housing

stable soil conditions and safe use.  Paved surfaces shall be

free of missing portions and major cracks, holes, tripping

hazards, spalling, dips or bulges.  Paving shall be free of

deterioration from moisture, decay or weathering.

Note:  Major dips may be an indication of a problem with

subsurface systems and should be investigated.

10-5.  FENCING AND RAILING.  Fencing and railing shall perform their

intended function in an efficient manner.  Fencing shall not

create hiding places for potential criminals.  Fencing and

railings shall be stable with securely anchored members.

*10-6.  FINISHES.

a.  Paint.  Paint to be applied on the interior and exterior

of buildings shall not have a lead content greater than

the amount permitted by 24 CFR Part 35. HUD regulations

24 CFR 968.9 require that all surfaces tested and found

to have a lead content higher than permitted are to be

treated as prescribed in the regulation to eliminate the

hazards of lead-based paint poisoning.  See 968.9(e) (3).

Paint used on building exteriors shall be of a durable

weather-resistant type to prevent excessive failure and

defects.  Painted surfaces shall be free of chalking,

fading and/or blistering.

b.  Millwork.  Millwork, including windows, doors, trim,

closets, etc., shall be sanded, primed and finish

painted to prevent splintering or water infiltration.

Millwork used on painted building exteriors shall be

finished to prevent moisture penetration.  Millwork shall

be in safe and sound condition, stable and anchored as

required.  Millwork shall be free of splintering, water

penetration, material deterioration, or the presence of

termites.

c.  Interior Areas Subject to Moisture and Water.  In areas

subject to water or moisture (e.g., kitchens, bathrooms

and laundries), wall, floor and ceiling finishes shall be

resistant to water, moisture, and damage from grease,

detergent and normal household chemicals.

10-3

2/88

d.  Wall and Ceiling Finishes.  Before finishing, it shall be

determined that walls and ceilings are stable, anchored

as required and free of moisture penetration.  Walls and

ceilings shall be free of holes, cracks, missing

portions and material deterioration.  Panels shall not

sag, buckle or delaminate.  All tile shall be secured and

silicone caulked.

(1)  Public Areas.  Wall and ceiling finishes used in

public and highly trafficked areas shall be of the

kind and quality to provide durability and

reasonable resistance to abuse and graffiti.  When

brittle or otherwise easily vandalized wall finishes

are repeatedly abused, they shall be replaced with a

vandal-resistant finish material up to 7 ft. above

the floor.  New or replacement finishes shall be

hard-wearing, resistant to vandalism and graffiti

and relatively easy to maintain.  Plastic

laminates, glazed tile epoxy coating or other

equivalent should be considered because of their

expected life, chemical resistance and easy

maintenance.

(2)  Bathrooms.  Wall finishes at bathtubs and showers

shall be water-impervious.  Showers and bathtubs

with showers shall have ceramic tile, porcelain

steel panel or reinforced fiber glass panel finish

on adjacent walls up to 6 ft. above the finished

floor.  Bathtubs without showers shall have a

minimum of 4 rows of ceramic tile around the top of

the bathtub.  The finished product shall meet the

standard in paragraph 5-4c.  Gypsum board used as

backing for wainscot in showers or tub enclosures

shall be water-resistant.  Insulating foil-backed

wall board shall not be used.

(3)  Elderly Projects.  Abrasive wall products shall not

be used in elderly projects.

e.  Floor Finishes.  Before finishing, it shall be

determined that they provide safe support for all

intended loads and are reasonably free of vibration or

deflection.  When exposed to water, all floors shall

drain to maintain safe conditions at all times.  Carpet

shall only be provided in projects or dwelling units

occupied by the elderly or the

2/85

10-4

___________________________________________________________________________

W-3-1, W-2(H), R-1, R-3-1(PIH), R-3-2, R-6, R-6-1,

R-6-2, R-7, R-7-1, R-7-2, R-9, R-9-1, 138-2, 138-7

W-3-1    Directives Management Officers--Headquarters and Regions,

library, ACIR (Advisory Commission on Intergovermental

Relations)

W-2      HQ Office Directors, Special Assistants,

those reporting directly to  Assistant Secretaries

R-1      Regional Administrators, Deputy Regional Administrators

R-3-1    Bulk shipment to Regional Offices for selective

distribution by program area

R-3-2    Directors, Administrative Services Divisions and

Administrative and Management Services Divisions, and

Administrative staffs

R-6      Category A offices - Office Managers and Deputy Office

Managers

R-6-1    Category A offices - bulk

R-6-2    Category A offices - Division Directors

R-7      Category B offices - Office Managers and Deputy Office

Managers

R-7-1    Category B offices - bulk

R-7-2    Category B offices - Division Directors

R-9      Directors and Deputy Directors, Offices of Indian Programs

R-9-1    Offices of Indian Programs - bulk