APPENDIX 3

APPENDIX 3.  PHYSICAL NEEDS ASSESSMENT - COMPREHENSIVE

IMPROVEMENT ASSISTANCE PROGRAM

SECTION 1.  INTRODUCTION

1.  OBJECTIVES.  This Appendix contains instructions for use of

the Survey Instrument in conducting the required physical

needs assessment under the Comprehensive Improvement

Assistance Program (CIAP).  Data derived from this assessment

will permit completion of the Form HUD-52825, Comprehensive

Assessment/Program Budget (See Appendix 10, CIAP Handbook

7485.1 REV-2), and will serve as a verification of the

project's needs during the Joint Review.  This Survey

Instrument is completed by the PHA before the Joint Review.

SECTION 2.  INSTRUCTIONS

2.  ORGANIZING FOR THE SURVEY.  The physical needs assessment

shall be undertaken in preparation for the Joint Review.

Performance of the assessment, especially in large PHAs with

sufficient staff, may be accomplished via a team approach or

separate teams doing the assessment and the energy audit.

The PHA should organize the assessment by:

a.  Reviewing the Survey Instrument to become familiar with

the types of information requested.  Disregard items

which do not apply to the particular project's physical

characteristics.

b.  Identifying personnel responsible for conducting the

assessment.

c.  Obtaining information from prior surveys that might have

some of the data required for the physical needs

assessment.

d.  Recording information from project architectural and

mechanical drawings and prior relevant surveys onto the

Survey Instrument.

3.  SCOPE OF SURVEY.  Judgment must be used in determining the

extent of the survey.  For example, when conditions are known

to be similar in all units, a sampling of dwelling units may

be surveyed.  When variations are substantial, a 100% survey

shall be conducted.  In determining the extent of the survey,

the following precepts shall apply:

Page 1                 2/85

APPENDIX 3

a.  Nondwelling Spaces.

(1)  If possible, survey all nondwelling spaces, but at

least 50% if conditions are known to vary; or

(2)  If all such spaces are known to be in the same

condition, survey at least 20% of the total.

b.  Dwelling Units.

(1)  Of all units are in similar condition, survey 10%

or a statistically valid sampling of the dwelling

units.  Include in the sampling each type of unit

size, e.g., one-, two-, and three-bedroom, and

survey  typical unit types in typical buildings.

(2)  Survey those units which are more subject to

architectural stress, such as:

(a)  ground floor units;

(b)  units underneath the roof;

(c)  units adjacent to elevator cores; and

(d)  units on the sides of the building which

receive the most weathering.

4.  SURVEY INSTRUMENT.  The Survey Instrument is divided into

sixteen components which simulate the way in which an

inspector would "walk though" a public housing project.

These components are listed at the beginning of the Survey

Instrument.

a.  Abbreviations and Symbols.  The Page content for

nondwelling spaces is similar for the first fourteen

components of the survey.  A sample page (Figure 1) is

provided with both abbreviations and symbols noted.

These are explained as follows:

2/85                            Page 2

APPENDIX 3

(1) The four categories of need are noted by the

following symbols:

o    E, Emergency - health and safety items requiring

immediate correction.

o    En, Energy items which are energy related and

may be a part of a cost-effective energy

conservative opportunity (ECO) selected from HUD

publication, Energy Conservation for Housing:

A Workbook (HUD-PD-700(3) or other HUD-approved

energy audits.  In such cases, Handbook 7485.2

REV-2 should be referenced as well as the

completed energy audit to determine the extent

to which an actual ECO is involved.

o    S, Security - project specific items which are

related to tenant and project security.

o    Ps, Project Specific - other project specific

items which are necessary for long-term

viability.

(2) Each condition is keyed by the surveyor to the

applicable four categories of need.

(3) The Survey Instrument is keyed to the applicable

section of this Handbook, e.g., in Figure 1, ROADs,

GENERAL 3-2 references Paragraph 3-2 of the Handbook.

(4) The box S, Satisfactory, is to be checked by the

surveyor if the condition of the survey item is

satisfactory, requiring no remedial action.

b.    Page Structure Dwelling Unit Inventory.  The dwelling

unit survey consists of two parts: the Project-Wide

Dwelling Unit Inventory; and the Dwelling Unit Survey.

(1) Project-Wide Dwelling Unit Inventory provides for the

collection of information about the generally

prevalent characteristics and conditions of all the

dwelling units and serves as a one-time general

evaluation about such features.  The profile is

completed one time for each bedroom size and

different configuration.  Conditions surveyed are of

the type that apply to all dwelling units of the

given size and type.  Hence, there is no need to

survey all dwelling units to ascertain the presence

of items.  For example, if closets or kitchen

2/85                            Page 4

storage facilities are not equipped with doors, there

is no need to complete this question for all dwelling

units.  The findings of this survey must be expanded,

however, to cover all of the bedroom unit types

surveyed.

(2) Dwelling Unit Survey covers any problem items

specific to the condition of the individual dwelling

unit being surveyed which is not generic to the class

of dwelling units.  In the example above, one would

be concerned with the condition of closets and of

kitchen storage facilities, but not the absence of

doors.

6.  PHYSICAL NEEDS ASSESSMENT ANALYSIS.  The following steps must

be taken after the physical needs assessment, including the

energy audit, is completed.

a.  Quantify and Expand.  The analysis of the assessment and

energy audit should be done jointly after quantities of

work items have been summarized on each individual survey

summary sheet (Figure 2).  This is so that a joint agenda

may be prepared for subsequent costing and so that

related work items may be combined into single tasks.

(1) If less than 100% of the project was Surveyed for

certain work items, the quantity required for

remedial action should be expanded to cover the

entire project.  For example, in a 50% survey of 100

units:

Survey

Remedial Action        Coverage   Quantity Observed

Patch and Plaster        50%      50 bathrooms x 25

25 sq. ft. of bathroom            sq. ft. or 1250 sq.

walls                             ft.

If all bathrooms are likely to require the above

remedial action, the expansion to 100 units is

calculated dividing the quantity observed (1250 sq.

ft.) by the percentage of survey coverage (.50).  The

quantity eyed is 2500 sq. ft.  The expansion should

be written on the survey.  Figure 2 shows addition

samples.

Page 5                   2/85

FIGURE 2, APPENDIX 3

SAMPLES OF EXPANDED SURVEY ITEMS

250 Unit Project in 5 Buildings

Remedial         Survey       Quantity        Quantity   Quantity

Action           Coverage     Observed        Required   Expanded

Install parking     50%         20LF           20/.50        40LF

lot wheelstops

Install closet      30%         75             250 or         250

doors in dwelling                              75/.30

units-all

Install closet     100%         50             50              50

doors in units -

building 1 only

Recaulk around      30%         9 LF           9x75/.30      2250LF

bathtubs                                       or 9x250

Replace screen      30%         40             40/.30         133

windows

2/85                            Page 6

APPENDIX 3

b.  Organize.  The organization of the quantified results should

follow the 16 general areas of the Survey Instrument.  PHAs

will be able to identify comprehensive systems of

improvements, which include the results of the energy audit,

in the following fashion on the suggested Project Cost

Estimate Worksheet, Figure 3:

(1) Identify project building component, i.e., site,

building envelope, etc.

(2) Identify itemized remedial action, i.e., install,

repair, or replace.

(3) Transfer units and quantities onto the Project Cost

Estimate Worksheet, i.e., linear feet, square feet,

each, etc.

(4) Check from survey(s) if the remedial action is an

emergency, energy, security or project specific item. If

it is an energy item, record the cost of installation in

the cost column and check the column marked EN, as well

as the other columns for which the item is appropriate.

Note that the cost of an energy item will come from the

energy audit.

c.  Estimate Costs.  With the above steps completed, projects

should have a composite listing of remedial actions and

quantities needed to bring the project up to the mandatory

standards.  This listing from the Project Cost Estimate Work-

sheets must be cost estimated, using the category of

improvements for which funds are to be requested on Form HUD-

52825.  The following steps are suggested to obtain total

costs:

(1) Ascertain PHA in-house capability.

(2) If there are no in-house cost estimation procedures,

obtain estimates from locally recognized cost indices,

local cost estimators or from any source commonly used

and approved by the PHA.

(3) When a source has been identified, note that the

estimate must include the total or unit cost of an item

installed.  Note where force account labor is used,

materials, labor, supervision and job-related costs shall

apply.

Page 7

2/85

APPENDIX 3

(4) Since all cost estimates must specify materials, in the

event that material specifications exceed the mandatory

standards, due to current materials, security or

individual project requirements, provide a justification

with the Final Application.

(5) Record the unit cost and total cost in the appropriate

columns, making sure that appropriate category columns

are checked, i.e., E(emergency), En(energy), S(security),

Ps(project specific).  Some items may be appropriate for

more than one category column.

(6) Cost estimates are to include all needss for which funds

are required.  This includes demolition, construction,

finishing, and related repairs thereto as well as

installation, materials, labor and supervision.  The

Project Cost Estimate Worksheet and supporting

documentation shall be presented at the Joint Review.

The PHA shall indicate when labor costs are based on

force account.

(7) Whenever possible, obtain costs for systems of

improvements which may allow quantity cost savings on

packages which will remedy several problems with a

single cost and material.

(8) Total all costs with a check in the E(emergency) column.

To prevent double counting, note whenever the item is

also an ECO.  This may be noted as a subtotal, i.e.,

Emergency - Energy.

(9) Total all costs with a check in the PS(project specific)

column.  Avoid double counting for ECOs; utilize sub-

totals, i.e., Project Specific - Energy.

(10) Total all remaining costs which have no check in the PS

or E columns, again insuring against double counts by

noting subtotals.

(11) Record any remaining ECO costs deemed cost-effective

from the energy audit and not recorded in any of the

above totals.

(12) Add all of the above to obtain the grand total cost

estimate for the project.  Figure 4 demonstrates this

costing procedure.

Page 9

2/85