- S. Department of Housing and Urban Development

Office of Public and Indian Housing

_________________________________________________________________

Special Attention of:                 NOTICE PIH 99-31 (HA)

Secretary Representatives;

State/Area Coordinators;              Issued:  July 29, 1999

Public Housing Directors;             Expires: July 31, 2000

Public Housing Agencies;              ___________________________

Resident Management Corporations      Cross Reference

(RMCs); Resident Councils (RCs);

Tribally Designated Housing

Entities (THDEs)

Subject: Guidance for Housing Agencies when handling Asbestos Containing Materials (ACMs) in Public Housing Modernization or Demolition

PURPOSE:

The purpose of this notice is to provide guidance to public

housing agencies/Tribally Designated Housing Entities

(THDEs), thereafter called HAs, regarding their legal obligation

and compliance responsibilities under local, State and Federal

requirements for handling asbestos-containing-materials (ACMs) in

public housing developments undergoing modernization or

demolition.

Undertaking modernization actions where asbestos is present is

subject to numerous and growing regulations at all levels of

government.  Careless or illegal handling of ACMs can subject

modernization contractors, workers, residents and visitors to

health hazards.  Improper or unlawful handling of asbestos can

place the contractor and HAs and property owners who are not

housing agencies in a position of serious civil and even criminal

liability.

ASBESTOS AND ITS USES:

Asbestos is a generic term that refers to a family of mineral

mineral silicates that separate into thin, needle like but

extremely strong fibers.  The unique characteristics of asbestos

have made it ideal for numerous construction products and uses,

including sound and fire proofing and insulation.  Asbestos

cannot be destroyed and it is not biodegraded easily.

Asbestos materials fall into two broad categories:  Friable and

non-friable.  Friable ACMs are products, which, when dry, can be

crumbled, pulverized, disturbed, punctured and easily reduced to

powder by hand pressure.  Friable ACMs emit fibers into the

atmosphere with relative ease when disturbed.  Non-friable ACMs

are bound into some type of hard matrix such as roofing, siding,

-2-

or flooring, and generally do not escape under ordinary use.

However, non-friable ACMs may become friable due to extreme

disturbance or deterioration.

When friable ACM is disturbed, microscopic asbestos fiber dust is

released into the air.  These fibers, much smaller and more

buoyant than ordinary dust particles, float almost indefinitely

and must be removed from the air by the use of a specialized High

Efficiency Particulate Air (HEPA) filter.  It is estimated that

two-thirds or more of the products made with asbestos are in the

air and are easily inhaled or ingested. They can only used in the

construction industry.  Most housing and buildings constructed

before 1979 will probably contain some asbestos.  Although the

building/construction industry generally discontinued using ACM

insulation in the mid-1970s, other products containing asbestos

may still be used.

FEDERAL ASBESTOS RULES:

There are no Federal regulations that require HAs to inspect,

survey, test or assess for ACM.  However, there are Federal

requirements that regulate:

1.  Worker exposure to asbestos;

2.  Procedures for abating asbestos when a building         under goes renovation or demolition; and

3.  Disposal of asbestos containing materials.

The Environmental Protection Agency (EPA) and the Occupational

Safety and Health Administration (OSHA) have been concerned with

the potential health hazards associated with exposure to asbestos

since the early 1970s.  They have major responsibility for

Federal regulatory control over exposure to asbestos.  The EPA

regulates asbestos procedures for renovation (modernization and

other actions that require removal of ACM) or demolition

(including partial demolition) actions under the Clean Air Act

and require notification and strict work practices for asbestos

handling, removal, storage and transport under 40 CFR Part 61,

Subpart M and 40 CFR Part 763.

OSHA regulates worker protection standards and exposures.  For

details regarding the OSHA regulations, refer to 29 CFR 1910.1001

(nonconstruction) and 1926.1101 (construction).  The EPA also

regulates worker protection of certain State and local government

employees not covered by the OSHA or comparable State regulation

(see 40 CFR Part 763, Subpart G).

-3-

Other Federal authorities or agencies that regulate asbestos

include:  The Clean Water Act under which the EPA has set

standards for asbestos levels in the effluent of navigable water;

the Mine safety and Health Administration which oversees the

safety of workers involved in the mining of asbestos; the

Consumer Product Safety Commission; the Food and Drug

Administration; and the Department of Transportation.

STATE AND LOCAL RULES:

At the State and local level, a wide variety of asbestos

regulations and guidelines have been established.  Many States

have either adopted the Federal rules or created something

comparable or more stringent.  Some State and local laws, like

the Federal ones, require the training and licensing of

contractors, inspectors, laboratories, project safety monitors

and asbestos abatement actions.  They may also require some of

the following:  (a) The inspection of buildings for friable ACMs

when renovation or modernization work will be performed and (b)

specific procedures for renovating, repairing, and abating

buildings when ACMs are disturbed.  HAs and contractors are

responsible for finding out which applicable Federal, State and

local law will apply to their situation and the ACMs in the

buildings they control.

FUNDING REQUIREMENTS:

Under the Comprehensive Grant Program (CGP), the Comprehensive

Improvements Assistance Program (CIAP), and the HOPE VI Program,

testing for and abatement of asbestos is an eligible Program

expenditure.

Public and Indian Housing requires compliance with Federal, State

and local laws and codes.

ASBESTOS ASSESSMENT/REMOVAL/MAINTENANCE:

Except for specific items like boiler and pipe

insulation/wrapping, the only sure way to know whether  or not

asbestos is present is to have an asbestos assessment

performed.  The assessment (inspection, test, or survey) should

be conducted by a National Standards for Hazardous Air Pollutants

(NESHAP) certified asbestos consultant or State or local

equivalent, architect, consulting engineer, State-certified

inspector, certified industrial hygienist, or EPA certified

company experienced in asbestos assessment.

To make certain that workers, residents, and the general public

are not exposed to asbestos as a result of

modernization/renovation or demolition work at a site, an

-4-

assessment should be done before such action starts.  The actions

taken, after it is determined that a building or property

contains ACM, depends upon the condition of the asbestos.  If

asbestos is present, an assessment may protect the program

participant from being in violation of the law, or subject to

fines or lawsuits.  When workers are abating a building, program

participants (HAs, contractors, residents) are urged to ask

questions and get as much information as possible on their

obligations and responsibilities under the law.  OSHA regulations

require resident relocation whenever there is a chance of

exposure to asbestos during renovations.

Program participants can be cited for violation of the EPA and

the OSHA regulations at the Federal level which carry fines for

asbestos violations.  Depending on the State and local

requirements, there may also be State/local fines where “illegal”

asbestos abatement is occurring.

All demolition activities require EPA notification, including

those that do not involve ACM.  Also, EPA has developed special

guidance on how to avoid the asbestos Clean Air Act requirements

by taking specialized measures during ACM removal activities

(such as for removing asbestos containing floor tiles).

A significant quantity of asbestos-containing waste may be

generated during removal of friable asbestos material from

buildings.  The EPA regulations address removal of friable

asbestos material prior to the demolition or renovation of

buildings.  Removal should be considered for materials that may

potentially become friable during demolition/ rehabilitation

activities.

An HA may reduce its asbestos related liability by having

asbestos assessment performed even when it is not contemplating

modernization and by then developing an ACM Operation and

Management (O&M) Plan for any identified ACM.

GUIDANCE/CONTACTS:

The responsibility for knowing about and complying with Federal,

State and local asbestos abatement requirements rests with the

HUD program participant.  PHAs should be aware that EPA, OSHA and

States have regulations governing proper maintenance of ACMs.

Further information can be obtained by consulting the EPA and the

OSHA Office in your area or HUD Field Office Environmental

Officer.  Below you will find a brief list of agency contacts.

-5-

It is important to remember that the information needed and the

HUD program participant’s responsibilities under the law will

depend on the participant’s relationship (HA, contractor,

residents, etc.) with the property.

/s/ Deborah Vincent for

____________________________

Harold Lucas

Assistant Secretary for

Public and Indian Housing