Attachment 7, Page 1

WINTERIZATION SERVICE REQUIREMENTS AND COST SCHEDULE

In jurisdictions where winterization is required, all properties must be winterized between October 1 and March 31.  Generally, properties should not be winterized between April 1 and September 30.  Exceptions to these winterization time periods are specified in this attachment.  Properties should be winterized only once.  The property may be re-winterized only if the initial winterization is deemed to be no longer effective and the mortgagee obtains prior approval from the M&M contractor.  The winterization process must include a thorough and complete draining of all plumbing and heating systems unless otherwise directed by the City/State variation.

M&M contractors should be contacted for directions on maintaining water services to houses where a community water service is involved and utility services to townhouses, duplexes, attached dwellings in PUDS, and condominiums where such services must be left on due to homeowners association or party wall requirements.  In condominiums and attached dwellings in PUDS, water services and utilities should remain on if the systems are shared with other units.

DRY HEAT SYSTEMS – The water supply to the property should be shut off at the curb.  If curb shut off is not possible, the main interior water supply must be shut off.  In either case, the water meter must be disconnected and left in the property unless the City or County ordinance requires return of the meter to the water company to eliminate further water charges.  The disconnected feed pipe leading from the main water valve must be plugged.  Tags, labels, or warning signs must be affixed to all items winterized and must include the date of winterization and the name, address, and telephone number of the firm that performed the work.  For properties with wells, the pump (if not submersible) and tank must be drained. The hot water heater and all domestic supply and distribution piping must be thoroughly drained.  All faucet and valves must be opened in the process and then closed after draining is completed. Toilets must be cleaned before winterization materials are used.  Use of air pressure to clear the system or, in some cases, adding anti-freeze to the systems are both acceptable provided that the effect is prevention of freeze-ups.  Adequate amounts of anti-freeze are to be placed in all fixture traps including toilet tanks and bowls.  All winterization must be performed in accordance with state and local codes, ordinances and regulations.

WET HEAT, RADIANT, HYDRONIC OR HOT WATER BASEBOARD SYSTEMS

The winterization requirements outlined above for domestic water apply except that water should not be shut off to or in the property.  Test for system security.  Pressurize the system with an air compressor to 50 pounds and inspect for leakage.  System must hold air pressure with no leakage for a minimum of one-half hour.

Drain the boiler and all heating loops.  Repair, replace, or install (as necessary to comply with state health department requirements) a Reduced Pressure Zone (RPZ) valve.  Such a valve shall be installed between the furnace and the main feed water supply, adjacent to the furnace.

Attachment 7, Page 2

Fill the boiler and all heating loops with a non-toxic anti-freeze (propylene-glycol) solution designated for hot water heating systems (with anti-rust ingredient) tested to a freeze point of –40 degrees Fahrenheit, or lower.  Isolate loops or bleed in sequence to ensure proper flushing through the lines.  Bleed all vents in the system to ensure that all air has been voided from the system.

Return the heating system to normal operating temperature and pressure.  Check the entire system for proper operation.  Leave heat on.  Tape the furnace electrical switch to the “on” position.  Tape card above the switch stating, “Do Not Remove Tape.  Do Not Turn Off.”

Set the thermostat at 55-degrees.  In the event major repair or replacement would be necessary in order to make the heating system operational, contact the M&M contractor for instructions.

STEAM HEAT SYSTEMS – The winterization requirements outlined above for domestic water apply.  Determine if the system is operable and if there are any leaks.  Report this information to the M&M contractor.  In addition, the house boiler system must be thoroughly drained.  All radiator vents are to be opened in the process and bleeder pins must not be removed from the radiators.

SUMP PUMPS – Where there is an existing sump pump used to keep basements or crawl space dry, the electricity must remain on to assure that the property is not damaged by flooding.  Check to make sure the sump pump is operating.  Contact the M&M contractor for guidance if the existing pump does not work.

See Attachment 3 for instructions regarding swimming pools.

Attachment 7, Page 3

WINTERIZATION SERVICE COST SCHEDULE

| Philadelphia HOC | Dry Heat: 1 Unit | Dry Heat: Additional Units | Wet Heat: 1 Unit | Wet Heat: Additional Units | Radiant Heat: 1 Unit | Radiant Heat: Additional Units |
| --- | --- | --- | --- | --- | --- | --- |
| Connecticut | $100 | $35 / Unit | $135 | $70 / Unit | $135 | $115 / Unit |
| Delaware | $60 | $35 / Unit | $110 | $70 / Unit | $110 | $90 / Unit |
| District of Columbia | $60 | $35 / Unit | $110 | $70 / Unit | $110 | $90 / Unit |
| Maine | $100 | $35 / Unit | $135 | $70 / Unit | $135 | $115 / Unit |
| Maryland | $60 | $35 / Unit | $110 | $70 / Unit | $110 | $90 / Unit |
| Massachu-setts | $100 | $35 / Unit | $135 | $70 / Unit | $135 | $115 / Unit |
| Michigan | $55 | $25 / Unit | $70 | $55 / Unit | $70 | $60 / Unit |
| New Hampshire | $100 | $35 / Unit | $135 | $70 / Unit | $135 | $115/ Unit |
| New Jersey | $100 | $35 / Unit | $135 | $70 / Unit | $135 | $115 / Unit |
| New York | $100 | $35 / Unit | $160 | $70 / Unit | $135 | $115 / Unit |
| Ohio | $55 | $25 / Unit | $70 | $55 / Unit | $70 | $60 / Unit |
| Penn-sylvania | $60 | $35 / Unit | $110 | $70 / Unit | $110 | $90 / Unit |
| Rhode Island | $100 | $35 / Unit | $135 | $70 / Unit | $135 | $115 / Unit |
| Vermont | $100 | $35 / Unit | $135 | $70 / Unit | $135 | $115 / Unit |
| Virginia | $60 | $35 / Unit | $110 | $70 / Unit | $110 | $90 / Unit |
| West Virginia | $60 | $35 / Unit | $110 | $70 / Unit | $110 | $90 / Unit |

Note:  The cost of anti-freeze is included in the cost allowable.

Attachment 7, Page 4

WINTERIZATION SERVICE COST SCHEDULE

| Atlanta HOC | Dry Heat: 1 Unit | Dry Heat: Additional Units | Wet Heat: 1 Unit | Wet Heat: Additional Units | Radiant Heat: 1 Unit | Radiant Heat: Additional Units |
| --- | --- | --- | --- | --- | --- | --- |
| Alabama | $55 | $35 / Unit | $55 | $55 / Unit | $55 | $55 / Unit |
| Caribbean | N/A | N/A | N/A | N/A | N/A | N/A |
| Florida | N/A | N/A | N/A | N/A | N/A | N/A |
| Georgia | $55 | $35 / Unit | $55 | $55 / Unit | $55 | $55 / Unit |
| Illinois | $55 | $35 / Unit | $70 | $60 / Unit | $70 | $65 / Unit |
| Indiana | $55 | $35 / Unit | $70 | $60 / Unit | $70 | $65 / Unit |
| JacksonvilleFlorida | $55 | $35 / Unit | $55 | $55 / Unit | $55 | $55 / Unit |
| Kentucky | $55 | $35 / Unit | $55 | $55 / Unit | $55 | $55 / Unit |
| Mississippi | $55 | $35 / Unit | $55 | $55 / Unit | $55 | $55 / Unit |
| North Carolina | $55 | $35 / Unit | $55 | $55 / Unit | $55 | $55 / Unit |
| South Carolina | $55 | $35 / Unit | $55 | $55 / Unit | $55 | $55 / Unit |
| Tennessee | $55 | $35 / Unit | $55 | $55 / Unit | $55 | $55 / Unit |

Note:  The cost of anti-freeze is included in the cost allowable.

Attachment 7, Page 5

WINTERIZATION SERVICE COST SCHEDULE

| Denver HOC | Dry Heat: 1 Unit | Dry Heat: Additional Units | Wet Heat: 1 Unit | Wet Heat: Additional Units | Radiant Heat: 1 Unit | Radiant Heat: Additional Units |
| --- | --- | --- | --- | --- | --- | --- |
| Arkansas | $50 | $35 / Unit | $90 | $70 / Unit | $135 | $115 / Unit |
| Colorado | $50 | $35 / Unit | $90 | $70 / Unit | $135 | $115 / Unit |
| Iowa | $50 | $35 / Unit | $90 | $70 / Unit | $135 | $115 / Unit |
| Kansas | $50 | $35 / Unit | $90 | $70 / Unit | $135 | $115 / Unit |
| Louisiana | $50 | $35 / Unit | $90 | $70 / Unit | $135 | $115 / Unit |
| Minnesota | $50 | $35 / Unit | $90 | $70 / Unit | $135 | $115 / Unit |
| Missouri | $50 | $35 / Unit | $90 | $70 / Unit | $135 | $115 / Unit |
| Montana | $50 | $35 / Unit | $90 | $70 / Unit | $135 | $115 / Unit |
| Nebraska | $50 | $35 / Unit | $90 | $70 / Unit | $135 | $115 / Unit |
| New Mexico | $50 | $35 / Unit | $90 | $70 / Unit | $135 | $115 / Unit |
| North Dakota | $50 | $35 / Unit | $90 | $70 / Unit | $135 | $115 / Unit |
| Oklahoma | $50 | $35 / Unit | $90 | $70 / Unit | $135 | $115 / Unit |
| South Dakota | $50 | $35 / Unit | $90 | $70 / Unit | $135 | $115 / Unit |
| Texas | $50 | $35 / Unit | $90 | $70 / Unit | $135 | $115 / Unit |
| Utah | $50 | $35 / Unit | $90 | $70 / Unit | $135 | $115 / Unit |
| Wisconsin | $50 | $35 / Unit | $90 | $70 / Unit | $135 | $115 / Unit |
| Wyoming | $50 | $35 / Unit | $90 | $70 / Unit | $135 | $115 / Unit |

Note:  The cost of anti-freeze is included in the cost allowable.

Attachment 7, Page 6

WINTERIZATION SERVICE COST SCHEDULE

| Santa Ana HOC | Dry Heat: 1 Unit | Dry Heat: Additional Units | Wet Heat: 1 Unit | Wet Heat: Additional Units | Radiant Heat: 1 Unit | Radiant Heat: Additional Units |
| --- | --- | --- | --- | --- | --- | --- |
| Alaska | $135 | $135 / Unit | $465 | $465 / Unit | $465 | $465 / Unit |
| Arizona | $80 | $35 / Unit | $125 | $65 / Unit | $125 | $65 / Unit |
| California | $80 | $35 / Unit | $125 | $65 / Unit | $125 | $65 / Unit |
| Hawaii / Pacific Islands | N/A | N/A | N/A | N/A | N/A | N/A |
| Idaho | $110 | $55 / Unit | $175 | $65 / Unit | $175 | $65 / Unit |
| Nevada | $80 | $35 / Unit | $125 | $65 / Unit | $125 | $65 / Unit |
| Oregon | $80 | $40 / Unit | $125 | $65 / Unit | $125 | $65 / Unit |
| Washington | $80 | $40 / Unit | $125 | $65 / Unit | $125 | $65 / Unit |

Note:  The cost of anti-freeze is included in the cost allowable.

Attachment 7, Page 7

LOCAL VARIATION FROM GENERAL REQUIREMENTS

NORTHEAST

City / State

Connecticut

Variation From General Requirement

Winterize year-round regardless of the month the property is secured.

Type of Service:                                                                                Allowable Cost:

Sump Pump	            $ 25

City / State

# Maine / New Hampshire / Vermont

Variation From General Requirement

Winterize year round regardless of the month the property is secured, except for attached condominiums (e.g. townhouse, garden style, highrise).  Attached condominiums must be kept heated, thermostat at 55 degrees Fahrenheit.  Utility accounts must be transferred to the mortgagee, and upon conveyance transferred to the M&M contractor.  Do not turn off or remove sump pump.  See specific requirements on pages 19-20.  Two important points: (1) turn water off at the curb, and (2) the mortgagee will be held responsible for damages to plumbing and heating systems caused by untimely and/or improper maintenance.

Type of Service							Allowable Cost

Sump Pump Installation		$135                                     (if one had existed)

Pumping Water from basement		$110

(only with prior approval)

Attachment 7, Page 8

City / State

# Massachusetts

Variation From General Requirement

Winterize year round regardless of the month the property is secured.

Type of Service	Allowable Cost

Sump Pump		$25

Water well  (pump, tank and lines)		$25

City / State

# New York

Variation From General Requirement

Winterize year round regardless of the month the property is secured.

City / State

# Buffalo, New York

Variation From General Requirement

Winterize year round, regardless of the month the property is secured.  All water lines must be pressure tested for leaks and then completely blown out with compressed air.  The mortgagee/contractor should check with the local jurisdiction to determine if there are restrictions on the use of anti-freeze compounds.  The mortgagee must note in the comments section of Form HUD-27011A when curb shut-off is not possible and the reason it could not be shut off.  Pools - see lock change/securing requirements.

City / State

# Rhode Island

Variation From General Requirement

Winterize year round regardless of the month the property is secured.

Type of Service:                                                                                Allowable Cost:

Sump Pump	            $ 30

Attachment 7, Page 9

MID-ATLANTIC

City / State

# Delaware / District of Columbia / Maryland / Pennsylvania / Virginia / West Virginia

Variation From General Requirement

Winterize year round regardless of the month the property is secured.

City / State

# Pittsburgh, Pennsylvania

Variation From General Requirement

Winterize year round, regardless of the month the property is secured.  When winterizing all properties located in the following areas of Pennsylvania: City of Erie, Harborcreek Township, Lawrence Park Township Millcreek Township, and Wesleyville, the mortgagee/contractor must contact the following public authorities to have water turned off at the curb and to stop trash pickups:

Erie Water Authority P.O. Box 1729, Erie, PA 16507 Phone: (814) 455-1049 ext. 258

Erie Public Works Department

Municipal Bldg., Rm 504

626 State Street

Erie, PA

Phone: (814) 870-1450

Failure to do this will result in water, sewage and trash pickup services to continue and for charges to continue to accrue against the property.

MID-WEST STATES (in Philadelphia and Atlanta Homeownership Centers)

City / State

# Illinois / Indiana

Variation From General Requirement

Winterize year round, regardless of the month the property is secured.

Type of Service:                                                                                Allowable Cost:

Sump Pump		$  15

Pumping water from basement		$100

Attachment 7, Page 10

City / State

# Michigan

Variation From General Requirement

Winterize year round regardless of the month the property is secured.  Install new submersible sump pump if existing pump is inoperable or missing from the property.

Type of Service:                                                                                Allowable Cost:

Sump Pump		$  15

Sump Pump replacement		$225

Water meter removal		$  25

Pumping water from basement		$100

Posting signs / stickers for winterization		$  10

City / State

# Ohio

Variation From General Requirement

Winterize year round regardless of the month the property is secured.

Type of Service:                                                                                Allowable Cost:

Sump Pump		$  15

Pumping water from basement		$100

Attachment 7, Page 11

SOUTHEAST

City / State

# Alabama

Variation From General Requirement

Electricity and utilities are to be turned off or disconnected unless approval to keep them turned on is obtained by the M&M contractor.  Do not disconnect water meter.  Use only non-toxic biodegradable materials.  Toilets must be cleaned before winterization materials are used.

City / State

# Caribbean

Variation From General Requirement

Do not winterize.

City / State

# Florida

Variation From General Requirement

The water supply must be shut off at the curb.  Notify utility companies to cancel all services.  Do not drain the water heater or water lines.  No other winterization is required.  These services are a standard requirement and do not have an allowable charge.

City / State

Georgia / Kentucky / North Carolina / South Carolina / Tennessee

Variation From General Requirement

See specific requirements on page 21 of this attachment.

City / State

# Mississippi

Variation From General Requirement

See specific requirements on page 21 of this attachment.

Attachment 7, Page 12

PLAINS/SOUTHWEST/ROCKY MOUNTAIN STATES (Denver Homeownership Center)

City / State

# Arkansas / Louisiana

Variation From General Requirement

Winterization is required from November 1 through March 30.  Properties should be winterized only once unless otherwise directed by the M&M contractor.  Re-winterization may be required if the initial winterization is found unsatisfactory and the mortgagee obtains prior approval from the M&M contractor.

Type of Service:                                                                                Allowable Cost:

Sump Pump ( 1 Unit)		$10

Sump Pump ( 2 Units)		$20

Sump Pump ( 3 Units)		$30

Sump Pump ( 4 Units                                                                                      $40

City / State

Colorado / Montana / Nebraska / North Dakota / South Dakota / Utah / Wyoming /

Variation From General Requirement

Winterization is required from September 1 through April 30.  Properties should be winterized only once unless otherwise directed by the M&M contractor.  Re-winterization may be required if the initial winterization is found unsatisfactory and the mortgagee obtains prior approval from the M&M contractor.

Type of Service:                                                                                Allowable Cost:

Sump Pump ( 1 Unit)		$10

Sump Pump ( 2 Units)		$20

Sump Pump ( 3 Units)		$30

Sump Pump ( 4 Units)		$40

Attachment 7, Page 13

City / State

# Iowa

Variation From General Requirement

Winterization is required from September 1 through April 30.  Properties should be winterized only once unless otherwise directed by the M&M contractor.  Re-winterization may be required if the initial winterization is found unsatisfactory and the mortgagee obtains prior approval from the M&M contractor.  Follow the directions on Page 1 of this Attachment, modified as follows:

Leave thermostat at 55 degrees.

Hot Water/Steam Heating Systems:

Check pipes and radiators for leaks and repair as necessary and leave system properly operating.  If the property has a propane or oil system, put on "KEEP FULL" contract with a local supplier.

Slab Foundations:

Leave heating system on and in proper operation.  If the property has a propane or oil system, put on "KEEP FULL" contract with a local supplier.  Systems check of furnace, lights, water heater, etc., if utilities are on - not to exceed $10. Send report to M&M contractor with conveyance papers.  If hot water heating system, leave on in proper operating conditions.

Private Wells and Septic Tanks:

Wells - all pressure tanks should be drained.  If pump is surface mounted, drain pump housing, - if submersible, then disconnect the check valve and drain all pump, suction, and discharge pipes.  All fixtures should be winterized.  Septic Tanks - consult with local contractor.

Type of Service:                                                                                Allowable Cost:

Sump Pump ( 1 Unit)		$10

Sump Pump ( 2 Units)		$20

Sump Pump ( 3 Units)		$30

Sump Pump ( 4 Units)		$40

Attachment 7, Page 14

City / State

# Kansas / Missouri

Variation From General Requirement

Winterization is required from September 1 through April 30.  Properties should be winterized only once unless otherwise directed by the M&M contractor.  Re-winterization may be required if the initial winterization is found unsatisfactory and the mortgagee obtains prior approval from the M&M contractor.

Type of Service:                                                                                Allowable Cost:

Sump Pump ( 1 Unit)		$10

Sump Pump ( 2 Units)		$20

Sump Pump ( 3 Units)		$30

Sump Pump ( 4 Units)		$40

City / State

# Minnesota

Variation From General Requirement

Winterization is required from September 1 through April 30.  Properties should be winterized only once unless otherwise directed by the M&M contractor.  Re-winterization may be required if the initial winterization is found unsatisfactory and the mortgagee obtains prior approval from the M&M contractor.  See specific requirements on pages 22-24 of this attachment.

Type of Service:                                                                                Allowable Cost:

RPZ Valve installation - required on all

hot water systems		$210

Sump Pump ( 1 Unit)		$  10

Sump Pump ( 2 Units)		$  20

Sump Pump ( 3 Units)		$  30

Sump Pump ( 4 Units)		$  40

Utilities (utility charges are not                                                          Actual

included in the per-unit maximum)                                                    Cost

Trip fee for properties outside 8 county                                             $30

Twin-City metro area (Anoka, Carver, Dakota, Hennepin,

Ramsey, Scott, Washington, Wright Counties) where the

contractor must travel over 50 miles from base to property.

Attachment 7, Page 15

City / State

# New Mexico

Variation From General Requirement

Winterization is required from November 1 through March 30 only.  Properties should be winterized only once unless otherwise directed by the M&M contractor and / or the HUD office.  Re-winterization may be required if the initial winterization is found unsatisfactory and the mortgagee obtains prior approval from the M&M contractor.

City / State

# Oklahoma

Variation From General Requirement

Winterization is required from September 1 through April 30.  Properties should be winterized only once unless otherwise directed by the M&M contractor.  Re-winterization may be required if the initial winterization is found unsatisfactory and the mortgagee obtains prior approval from the M&M contractor.

Type of Service:                                                                                Allowable Cost:

Sump Pump ( 1 Unit)		$10

Sump Pump ( 2 Units)		$20

Sump Pump ( 3 Units)		$30

Sump Pump ( 4 Units                                                                                      $40

City / State

# Texas

Variation From General Requirement

Winterization is required from November 1 through March 30 only.  Air blow systems to clear all lines, drain water heater, open all faucets and add 2 cups of environmentally safe anti-freeze to all commodes and 1/2 cup to all sinks and showers.  Properties should be winterized only once unless otherwise directed by the M&M contractor.  Re-winterization may be required if the initial winterization is found unsatisfactory and the mortgagee obtains prior approval from the M&M contractor.

Attachment 7, Page 16

City / State

# Wisconsin

Variation From General Requirement

Winterization is required from September 1 through April 30.  Do not turn water off at curb. Properties should be winterized only once unless otherwise directed by the M&M contractor.  Re-winterization may be required if the initial winterization is found unsatisfactory and the mortgagee obtains prior approval from the M&M contractor.

Type of Service:                                                                                Allowable Cost:

Sump Pump ( 1 Unit)		$  10

Sump Pump ( 2 Units)		$  20

Sump Pump ( 3 Units)		$  30

Sump Pump ( 4 Units)		$  40

Pumping water from basement

(only with prior approval)                                                                               $100

WEST

City / State

# Alaska

Variation From General Requirement

Properties should be winterized (with heat left on, thermostat set at 65 degrees Fahrenheit) ALWAYS, regardless of the time of the year, and should be conveyed with water, gas, and utilities ON.  Propane and fuel oil delivery services shall remain on automatic fill.  The servicing mortgagee is hereby instructed and authorized to switch utilities into HUD's name effective on the date of conveyance. Utilities shall remain in the servicing mortgagee's name until conveyance.  Water service may be turned off at the street (key box) only if the property has a forced air or electric heating system and the domestic water system has been properly winterized. See pages 25 through 32 for additional specifications, checklist and certifications.

City / State

# Arizona, California

Variation From General Requirement

Winterize properties above 2,000 feet only.

Attachment 7, Page 17

City / State

# Hawaii / Pacific Islands

Variation From General Requirement

Do not winterize.

City / State

# Idaho

Variation From General Requirement

Maximum indoor temperature is 55 degrees.  See pages 25-32 of this attachment for additional specifications, checklist and clarifications.

City / State

# Nevada

Variation From General Requirement

No winterization except for Pahrump zip code 89041 and Mt. Charleston zip code 89124.  All other winterization on a case-by-case basis.

City / State

# Reno, Nevada

Variation From General Requirement

Winterize from September 15 through April 15.  Use "non-toxic" anti-freeze.  In cases such as condominiums or PUD complexes that have no individual curbside shutoffs and no main shut off in the individual units, maintain the heat at 55 degrees Fahrenheit.

Attachment 7, Page 18

City / State

# Oregon

Variation From General Requirement

Winterize from November 1 through March 31 only.  Maximum indoor temperature is 55 degrees.  See pages 25-32 of this attachment for additional specifications, checklist and clarifications.

City / State

# Washington

Variation From General Requirement

Maximum indoor temperature is 55 degrees.  See pages 25-32 of this attachment for additional specifications, checklist and clarifications.

Attachment 7, Page 19

WINTERIZATION SERVICE REQUIREMENTS AND COST SCHEDULE

WINTERIZATION SPECIFICATIONS FOR MAINE, NEW HAMPSHIRE & VERMONT

Properties must be winterized on a year round basis.  Properties should be winterized only once.  The property may be re-winterized only if the initial winterization is deemed to be no longer effective and the mortgagee obtains prior approval from the M&M contractor.

Domestic Water

On all properties test for system integrity.  Turn the water on, inspect for leakage, and shut down. If problems are noted, inform the M&M contractor at time of conveyance.

If the water supply source is a public system, have the utility turn off the supply at the street. Turn off the water inside the house.  Do not cut any water lines or remove water meters.  (A non-reimbursable charge may be made by the water utility for unauthorized water meter removal).  Most water departments must be notified when water is turned off so that a final meter reading can be done.  Be sure to do this.

If the water supply is a private well, turn off the well at the breaker panel and tape off the breaker.  Disconnect the water supply line between the house and pressure tank and install a hose bib on the pressure tank side of the break.  Tag the hose bib "For Water Testing."

Drain and blow air through all domestic hot and cold water lines, the hot water heater, water softener tank, brine tank, well water storage tank and/or the well water captive air bank, and dishwasher, if any.  Pour one-quart maximum of non-toxic antifreeze (propylene-glycol) into each drain: tubs, shower, laundry, floor, sinks, etc.

Remove water from toilet tank and bowl.  Pour a minimum of one gallon of non-toxic antifreeze (propylene-glycol) into each toilet bowl and a small amount of antifreeze in tanks if the water was not completely removed.

Tape all faucets to the open position.  Mark all drains, fixtures, appliances, devices in the system with the statement "Winterized **** Do Not Use", and date.

Attachment 7, Page 20

WINTERIZATION SERVICE REQUIREMENTS AND COST SCHEDULE

Dry Heat System

The heating system should be operational.  In the event major repair or replacement would be necessary in order to make the heating system operational, contact the M&M contractor for instructions.

Wet Heat (Hydronic or Hot Water Baseboard) Systems

The winterization requirements outlined above for domestic water apply.  Test for system security.  Pressurize the system with an air compressor to 50 pounds and inspect for leakage.  System must hold air pressure with no leakage for a minimum of one-half hour.

Drain the boiler and all heating loops.

Fill the boiler and all heating loops with a non-toxic antifreeze (propylene-glycol) solution tested to a freeze point of -40 degrees Fahrenheit, or lower.  Isolate loops or bleed in sequence to ensure proper flushing through the lines.  Bleed all vents in the system to ensure that all air has been voided from the system.

Steam Heat Systems

Determine if the system is operable and if there are any leaks.  Report this information to the M&M contractor at the time of conveyance.  Then shut down the heating system. Pressurize the system with an air compressor to 50 pounds and inspect for leakage.  System must hold air pressure with no leakage for a minimum of one-half hour.  Note: A steam heat system will have radiators with valves at the bottom of the radiator; a careful determination as to whether the system is hot water or steam must be made.

Sump Pumps

Where a sump pump is used to keep basements or crawl space dry, check to make sure the sump pump is operable and operating to assure that the property is not damaged by flooding.  If sump pump is missing from its original placement or is non-operational, install new sump pump and verify operation to assure that the property is not damaged by flooding.

Utilities

Utility accounts must be in the mortgagee's name until conveyance of the property to HUD.  It is the mortgagee's responsibility to transfer the account to the M&M contractor.  In all cases, the mortgagee should pay the bills and obtain reimbursement for utility costs that cover the period from the start date to the conveyance date.

Attachment 7, Page 21

WINTERIZATION SERVICE REQUIREMENTS AND COST SCHEDULE

WINTERIZATION SPECIFICATIONS FOR SOUTHEAST / CARIBBEAN OFFICES

1.	Disconnect main water line at property side of meter. (Not applicable for State of Tennessee.)

2.	Apply air pressure to water lines sufficient to remove all water.

3.	Remove water from water heater, traps, and water closets.

4.	Cap disconnected water line.

5.	Place non-toxic antifreeze (minimum of one gallon per living unit or sufficient amount to prevent pipes from freezing) in all traps, toilet bowls, and water closets.

6.	Failure to properly winterize property may result in mortgagee being liable for freeze damage.

7.	Attach a "WARNING" sign at the kitchen sink, bath facilities and water heater, that at a minimum includes the following information:

WARNING

THIS PROPERTY SYSTEM WINTERIZED ON ______/______/____________

BY (NAME OF FIRM): _____________________________________________

PHONE NUMBER: _________________________________________________

DO NOT USE PLUMBING SYSTEM

WATER TESTING MUST BE ACCOMPLISHED WITHOUT DE-WINTERIZING

SYSTEMS

Attachment 7, Page 22

WINTERIZATION SERVICE REQUIREMENTS AND COST SCHEDULE

WINTERIZATION SPECIFICATIONS FOR MINNESOTA STATE OFFICE

Properties must be winterized from September 1 through April 30.  Properties should be winterized only once.  The property may be re-winterized only if the initial winterization is deemed to be no longer effective and the mortgagee obtains prior approval from the M&M contractor.

Domestic Water

On all properties test for system integrity.  Turn the water on to inspect for leakage, and shut down.  If problems are noted, inform the M&M contractor at the time of conveyance.

If the water supply source is a public system, have the utility turn off the supply at the street (key/stop box) if possible.  Turn off the water inside the house.  Do not cut any water lines or remove water meters.  (A non-reimbursable charge may be made by the water utility for unauthorized water meter removal).  Most water departments, particularly the City of Minneapolis, must be notified when water is turned off so that a final meter reading can be done.  Be sure to do this.  If the heating system is hydronic or hot water baseboard heating, the water must not be turned off (see below).

If the water supply is a private well, turn off the well at the breaker panel and tape off the breaker.  Disconnect the water supply line between the house and pressure tank and install a hose bib on the pressure tank side of the break.  Tag the hose bib "For Water Testing."

Drain and blow air through all domestic hot and cold water lines, the hot water heater, water softener tank, brine tank, well water storage tank and/or the well water captive air bank, and dishwasher, if any.  Pour one-quart maximum of non-toxic antifreeze (propylene-glycol) into each drain: tubs, shower, laundry, floor, sinks, etc.

Remove water from toilet tank and bowl.  Pour a minimum of one gallon of non-toxic antifreeze (propylene-glycol) into each toilet bowl and a small amount of antifreeze in tanks if the water was not completely removed.

Tape all faucets to the open position. Mark all drains, fixtures, appliances, devices in the system with the statement "Winterized **** Do Not Use", and date.

Attachment 7, Page 23

WINTERIZATION SERVICE REQUIREMENTS AND COST SCHEDULE

Dry Heat System

The heating system should be operating and the thermostat left at a 60-degree setting.  In the event major repair or replacement would be necessary in order to make the heating system operational, contact the M&M contractor for instructions.

Wet Heat (Hydronic or Hot Water Baseboard) Systems

The winterization requirements outlined above for domestic water apply except that water should not be shut off to or in the property.  Test for system security.  Pressurize the system with an air compressor to 50 pounds and inspect for leakage.  System must hold air pressure with no leakage for a minimum of one-half hour.

Drain the boiler and all heating loops.  Repair, replace, or install (as necessary to comply with state health department requirements) a Reduced Pressure Zone (RPZ) valve.  Such a valve shall be installed between the furnace and the main feed water supply, adjacent to the furnace.

Fill the boiler and all heating loops with a non-toxic antifreeze (propylene-glycol) solution designated for hot water heating systems (with anti-rust ingredient) tested to a freeze point of -40 degrees Fahrenheit, or lower.  Isolate loops or bleed in sequence to ensure proper flushing through the lines.  Bleed all vents in the system to ensure that all air has been voided from the system.

Return the heating system to normal operating temperature and pressure.  Check the entire system for proper operation.  Leave heat on.  Tape the furnace electrical switch to the "on" position. Tape card above the switch stating, "Do Not Remove Tape. Do Not Turn Off.”

Set the thermostat at 60-degrees.  In the event major repair or replacement would be necessary in order to make heating system operational, contact the M&M contractor for instructions.

Attachment 7, Page 24

WINTERIZATION SERVICE REQUIREMENTS AND COST SCHEDULE

Steam Heat Systems

Determine if the system is operable and if there are any leaks.  Report this information to HUD at time of conveyance.  Then shut down the heating system.  Pressurize the system with an air compressor to 50 pounds and inspect for leakage.  System must hold air pressure with no leakage for a minimum of one-half hour.  Note:  A steam heat system will have radiators with valves at the bottom of the radiator; a careful determination as to whether the system is hot water or steam must be made.

Sump Pumps

Where a sump pump is used to keep basements or crawl space dry, check to make sure the sump pump is operable and operating to assure that the property is not damaged by flooding.

Utilities

Electricity and gas (for heating) are to remain on.  Propane and oil service must be placed on automatic fill.  Billing to utilities should be in the mortgagee's name until conveyance of the property to HUD.  We will activate billing in HUD's name within 5 days after receipt of the copy of the Part A in the field office.  The mortgagee should not transfer billing to HUD.  In all cases, the mortgagee should pay the bill and obtain reimbursement for utility costs that cover the date of the conveyance until the end of that utility's billing cycle.

Attachment 7, Page 25

WINTERIZATION SERVICE REQUIREMENTS AND COST SCHEDULE

WINTERIZATION SPECIFICATIONS FOR NORTHWEST / ALASKA OFFICES

Winterization must include a thorough and complete draining of all plumbing and heating systems.  Use of air pressure in the system and antifreeze is acceptable provided the effect is to prevent freeze-ups.  The water supply to the property must be shut off at the curb.  If a curb shutoff is not possible, the main interior water supply must be shut off.  In either case, the water meter must be disconnected, but not removed from property, and the disconnected feed pipe leasing from the main water valve must be plugged.

A.	Turn off Water at Street:

1.  After winterization in all M&M contract areas, except Alaska.

2.  Alaska - Only turn water off at street if dry heat system is used, such as forced air heating system or electric heating system.

B.	Sump Pumps

Where a sump pump is being used to keep basements or crawl space dry, the electricity must remain on to assure that flooding does not damage the property.

C.	Dry Heat Systems

The hot water heater and all domestic supply and distribution piping must be thoroughly drained. All faucets and valves must REMAIN OPEN during the process, and then closed after draining is completed.  Adequate amounts of antifreeze are to be placed in all fixture traps, including toilet tanks and bowls.

Test domestic supply lines for system integrity.  Turn kitchen, bathrooms, laundry area, exterior and garage faucets to on position.

1.  Turn water on.  Water lines should be drained and blown free of water.  Inspect for leakage and shut down.  Blow entire system free of water, using no less than a 40-gallon tank with a minimum of 50 pounds pressure.  Pressure of line should not exceed 90 pounds.

Attachment 7, Page 26

WINTERIZATION SERVICE REQUIREMENTS AND COST SCHEDULE

Heating system must hold air pressure with no leakage for a minimum of one-half hour.

2.        Commode tanks - drain.

3.         Hot water tank - drain valve is to be in open position.

4.        Evaporative air conditioners and any existing water lines - drain.

5.        Water softener tank, brine tank, and the well water storage tank and/or the well water

captive air tank, dishwasher and washing machine - drain.

6.	Turn off water supply and disconnect (do not remove) at water meter.

a.	If the water supply source is city or community water, turn off the supply at the street (key box), if possible.  Turn off the water inside the house.  This may require installation of a gate valve.  Cut the supply line between the valve and the house.  Plug the cut supply line.

b.	If the water supply is a private well, turn off the well at breaker panel and TAPE OFF breaker.  Disconnect the pump at pump controller. Indicate on the breaker panel that the pump has been disconnected at the pressure tank and install a hose bib on the pressure tank side of the break.  Tag the hose bib "FOR HEALTH DEPARTMENT TESTING".

7.	Charge the entire domestic water system with nontoxic antifreeze (propylene glycol).

a.	Pour 2 quarts of non-toxic antifreeze (propylene 	glycol) into each drain.  This includes tubs, shower drains, laundry drains, sink drains, floor 	drains, lavatories, etc.

b.	Pour a minimum of one gallon of non-toxic antifreeze (propylene glycol) into each water closet tank and flush.

c.	Leave the outside sill cocks open.

Attachment 7, Page 27

WINTERIZATION SERVICE REQUIREMENTS AND COST SCHEDULE

d.	Tape all faucets to the CLOSED position.  Mark all 	drains, fixtures, appliances in system and devices with the statement "WINTERIZATION ****DO NOT USE."  Tape card to kitchen counter or other conspicuous 	place in the home containing the above information.

e.	Comply with all items on the following checklist, 	post one copy at the breaker panel and tape one copy to the kitchen counter and hot water tank:

(See pages 29-32 of this Attachment)

- Wet Heat Systems

The winterization requirements outlined above (domestic water supply) apply.  In addition, the house boiler system or heater must be thoroughly drained.  All radiator vents are to be opened in the process.  Bleeder pins must not be removed from the radiators.  Any radiant heat piping must be drained and blown dry by compressed air pressure and an adequate amount of antifreeze is to be put in the radiant piping.

- DO NOT TURN OFF THE WATER at the street (key box) if home heating system is

hydronic or hot water baseboard system.

2.         Confirm that fuel, (gas, oil, propane, etc.) electricity, and water are operational at the

property.

3.	Test for system integrity.  Turn the water on, inspect for leakage and shut down.                                              Pressurize system with air compressor to 50 pounds and inspect for leakage.  System must hold air pressure with no leakage for a minimum of one-half hour.  Under no circumstances is the water to be shut off at the street (key box).

4.	Test the non-toxic antifreeze (propylene glycol) for a freeze point of -30 degrees Fahrenheit, or lower. Ensure that glycol mixture is adequate to protect system from freezing.  If necessary, fill the boiler and all heating loops with a non-toxic antifreeze (propylene glycol) solution, isolating loops or bleeding in sequence to ensure proper flushing through the lines.  Verify that the existing solution has a freeze point of -30 degrees Fahrenheit or lower.  Bleed all the vents in the system to ensure that all air has been voided from the system.

5.	Ensure there is adequate protection to the boiler should the water pressure drop low.  If no protection exists, install a low water cut off switch on the boiler.

6.	Return heating system to normal operating temperature and pressure.  Check entire system for proper operation.

7.	Tape furnace electrical switch to the "ON" position.  Tape card above switch stating, "DO NOT REMOVE TAPE.  DO NOT TURN OFF."

Attachment 7, Page 28

WINTERIZATION SERVICE REQUIREMENTS AND COST SCHEDULE

E.	Swimming Pools/Hot Tubs/Jacuzzi

1.	Swimming Pools - Pump out pool leaving 4 feet of water in the deep end (this amount will vary depending on your elevation).  After pumping, apply chlorine to control the algae in the remaining water.  Circulate remaining water to mix chlorine.  Isolate and drain pumps and filters.  Remove all trash from interior of pool and haul away. **WARNING** Check local area standards before draining pool.  In area where there is a high ground water level, removing too much water may cause pool uplift and damage the pool.

If there is a pool cover present, secure and cover pool. Check with state and local codes and verify whether the pool must be covered or merely fenced in. If additional security is required, notify field office and request further instructions regarding any additional needed security for the tub/jacuzzi.

Attachment 7, Page 29

WINTERIZATION SERVICE REQUIREMENTS AND COST SCHEDULE

WINTERIZING CHECKLIST AND CERTIFICATION OF COMPLETION

NORTHWEST / ALASKA OFFICES

Property Address

This property has been winterized in accordance with U.S. Department of HUD, Northwest/Alaska Offices requirements.  All items checked have been performed on (date)______________________ to protect against damage by weather and the elements.

______Water lines drained and blown free of water.

______Dishwasher drained and blown out

______Hot water heater drained.

______Water shut OFF at meter or exterior service valve.

______Kitchen faucets turned to OFF position.

______Bathroom faucets turned to OFF position.

______Laundry area faucets turned to OFF position.

______Exterior faucets turned to OFF position.

______Non-toxic antifreeze in kitchen sink traps.

______Non-toxic antifreeze in bathroom lavatory and tub traps.

______Water closet reservoir drained.

______Non-toxic antifreeze in water closet and reservoirs.

______All hoses removed from exterior hose bibs.

______Water heater breaker SAFETY WIRED to OFF position.

______If hot water heating system is present, ensure integrity of the system.

______Drain all evaporative air conditioners and any existing water lines.

Attachment 7, Page 30

I hereby certify that this work has been completed in accordance with proper procedures and that

damage to the systems as a result of improper winterization shall be the responsibility of the contractor.

Contractor Signature	Contractor Address

Contractor Telephone	Date

WINTERIZATION SERVICE REQUIREMENTS AND COST SCHEDULE

(SAMPLE)

NORTHWEST / ALASKA CONTRACTOR CERTIFICATION

HYDRONIC (FORCED HOT WATER) HEATING - WATER LEFT ON

WINTERIZED BY

ADDRESS

PHONE

DATE

NOT RESPONSIBLE FOR SYSTEMS DEWINTERIZED BY PARTIES OTHER THAN OURSELVES.  HEALTH DEPARTMENT TESTING MUST BE ACCOMPLISHED WITHOUT DEWINTERIZING SYSTEMS. HYDRONIC WATER SYSTEM IS FUNCTIONAL AND WINTERIZED IN ACCORDANCE WITH THE SPECIFICATIONS OF THE DEPARTMENT OF HOUSING AND URBAN DEVELOPMENT, NORTHWEST / ALASKA OFFICES.

I HEREBY CERTIFY THAT THIS WORK HAS BEEN COMPLETED IN ACCORDANCE

WITH PROPER PROCEDURES AND THAT DAMAGE TO THE SYSTEMS AS A RESULT OF IMPROPER WINTERIZATION SHALL BE THE RESPONSIBILITY OF THE CONTRACTOR.

CONTRACTOR SIGNATURE

SERVICING MORTGAGEE SIGNATURE

Attachment 7, Page 31

WINTERIZATION SERVICE REQUIREMENTS AND COST SCHEDULE

(SAMPLE)

NORTHWEST ALASKA CONTRACTOR CERTIFICATION

WATER SYSTEM SHUT OFF

WINTERIZED BY

ADDRESS

PHONE

DATE

NOT RESPONSIBLE FOR SYSTEMS DEWINTERIZED BY PARTIES OTHER THAN OURSELVES. HEALTH DEPARTMENT TESTING MUST BE ACCOMPLISHED WITHOUT DEWINTERIZING SYSTEMS.  THE DOMESTIC WATER SYSTEM IS CUT OFF AND WINTERIZED IN ACCORDANCE WITH THE SPECIFICATIONS OF THE DEPARTMENT OF HOUSING AND URBAN DEVELOPMENT, NORTHWEST/ALASKA OFFICES.

I HEREBY CERTIFY THAT THIS WORK HAS BEEN COMPLETED IN ACCORDANCE WITH PROPER PROCEDURES AND THAT DAMAGE TO THE SYSTEMS AS A RESULT OF IMPROPER WINTERIZATION SHALL BE THE RESPONSIBILITY OF THE CONTRACTOR.

CONTRACTOR SIGNATURE

SERVICING MORTGAGEE SIGNATURE

Attachment 7, Page 32

WINTERIZATION SERVICE REQUIREMENTS AND COST SCHEDULE

(SAMPLE)

NORTHWEST / ALASKA CONTRACTOR CERTIFICATION

WATER AND ELECTRIC SERVICE BOTH DISCONNECTED

WINTERIZED BY

ADDRESS

PHONE

DATE

THIS PROPERTY HAS BEEN WINTERIZED IN ACCORDANCE WITH SPECIFICATIONS OF THE DEPARTMENT OF HOUSING AND URBAN DEVELOPMENT, NORTHWEST/ALASKA OFFICES.  DO NOT ATTEMPT TO RESTORE ELECTRICAL POWER OR WATER SERVICE UNTIL ALL PLUMBING AND ELECTRICAL SYSTEMS HAVE BEEN INSPECTED.

I HEREBY CERTIFY THAT THIS WORK HAS BEEN COMPLETED IN ACCORDANCE WITH PROPER PROCEDURES AND THAT DAMAGE TO THE SYSTEMS AS A RESULT OF IMPROPER WINTERIZATION SHALL BE THE RESPONSIBILITY OF THE CONTRACTOR.

CONTRACTOR SIGNATURE

SERVICING MORTGAGEE SIGNATURE