Lead Hazard Evaluation Notice

Property Address:				                                Date:

Evaluation Completed (circle one):    Paint Inspection     Paint Testing     Risk Assessment

Summary of Results:

____ No lead-based paint or lead-based paint hazards were found.

____ Lead-based paint and/or lead-based paint hazards were found. See attachment for details.

Person who prepared this notice:

Name: ________________________

Signature: _____________________

Date: _________________________

Company: _____________________

Street: ________________________

City & State: ___________________

Zip: __________________________

Phone # _______________________

If you have any questions please call Syracuse Lead Hazard Control Program at 448-8710

Lead-Based Paint

Risk Assessment / Inspection Report

For The Dwelling Located at:

123 Any Street

Syracuse, New York

Prepared For:

Ms. Betsy Mokrzycki, Program Manager

City of Syracuse Lead Program

201 East Washington Street

Syracuse, NY 13202

March 17, 2004

EXPIRATION DATE: March 10, 2005

GENERAL INFORMATION

The City of Syracuse Lead Program (USEPA Firm NY-07-022006-973) conducted a risk assessment / inspection for lead-based paint hazards at 123 Any Street in Syracuse, New York on March 10, 2004. John Doe (USEPA Risk Assessor NY-02-022004-789) and Jane Doe (USEPA Risk Assessor NY-05-022005-456) performed this Lead Based Paint Risk Assessment / inspection. The sample analysis was performed by: 123 Laboratories, Inc at 2345 W. Cary Street, Richmond, VA and can be reached by phone at (888)444-5555.

OWNER INFORMATION

| Address | Owner Information | Construction Date | Assessment Date |
| --- | --- | --- | --- |
| 123 Any Street Syracuse, NY | John Craig 123 Any Street Syracuse, NY 13205 Ph. 315-492-0556 | 1890 | March 10, 2004 |

An initial walk-through was conducted in the dwelling to determine the presence of suspect materials, which were accessible or exposed.  The information contained in this report has been collected in accordance with current regulations.

PURPOSE

This lead based paint hazard / inspection investigation was based upon chapter 5 and 7 of the United States Department of Housing and Community Development (HUD) Guidelines.  Reference levels are as listed below:

### DUST WIPE SAMPLES

Floors			40 µg/ft2

Window Sills		250 µg/ft2

XRF ASSAY						PAINT

1.0 mg/cm2					0.5% Lead by weight

# WATER SAMPLES

- 15 Parts per Billion (ppb)

- SOIL SAMPLES

Paving or removal criteria	5,000 ppm

Building perimeter/yard	1,200 ppm

Play/high contact areas	400 ppm

SCOPE OF SERVICES

A review of all available documentation concerning lead investigations and abatements conducted in the building was completed. A visual inspection for deteriorated paint was conducted. XRF assays and samples of settled dust were collected in accordance with current regulations to determine where lead paint exists and if any lead hazards were present in the home.  No soil samples were collected at this time due to snow coverage.

METHODOLOGY

Upon completion of the visual inspection, XRF assays and samples of settled dust were collected in selected locations and in a manner consistent with chapter 5 of the HUD Guidelines.

NARRATIVE

On March 10, 2004 The City of Syracuse Lead Program performed a lead based paint risk assessment / inspection at 123 Any Street in Syracuse, New York. Settled dust and XRF samples were taken in an effort to locate the possible sources of lead within and around the dwelling. No soil samples were taken at this time due to snow coverage.

The City of Syracuse Lead Program followed ASTM Standard Practice E1728, “Standard Practice for Field Collection of Settled Dust Samples Using Wipe Sampling Methods for Lead Determination by Atomic Spectrometry Techniques” for collection of dust wipe samples. Samples of settled dust were collected from within the units from floors and window sills that appeared to be the dirtiest and most accessible to children. In addition to these samples a field blank was sent to the laboratory for quality assurance and quality control purposes.

Within the dwelling the average of floor samples is 92.0µg/ft2 and the average of window sill samples is 369.0µg/ft2. This indicates that a lead dust hazard does exist at this time. It is our recommendation that all floors and all window sills be cleaned using the HUD three step cleaning method as described in Chapter 14-11 of the HUD Guidelines.

The City of Syracuse Lead Program tested 115 component surfaces from this location using a Niton XL000 XRF serial number U000NR0000. The XRF was calibrated following the manufacturer’s recommended protocol before and after the testing. Tested surfaces were selected in accordance with Chapter 7 of the HUD Guidelines. City of Syracuse lead Program, also considered the Onondaga County Health Department findings that are included in this report.

A detailed list of components that were tested by the XRF is provided in the following quick summary of analysis results with the samples exceeding the reference limit for lead in paint indicated in boldface type. In general, room one is defined as the first room or room equivalent directly accessed by the main entrance and rooms are labeled clockwise from that point. Kitchens are labeled as “kitchens” and bathrooms are labeled as “baths”. Also the “A” side of any apartment or building is the address side of the house and the sides are then labeled alphabetically going clockwise. If a given unit has two floors the numbering continues with the first non-bathroom on the left unless indicated otherwise on the floor plan.

A copy of this summary must be provided to new Lessees (tenants) and purchasers of this property under Federal Law (24 CFR part 35 and 40 CFR part 745) before they become obligated under a lease or sales contract. The complete report must also be provided to new purchasers and it must be made available to new tenants. Landlords (lessors) and sellers are also required to distribute an educational pamphlet approved by the U.S. Environmental Protection Agency and include standard warning language in their leases or sales contracts to ensure that parents have the information they need to protect their children from lead-based paint hazards

Any lead related work that involves the removal or disturbance of the leaded materials identified in this dwelling must be done in compliance with lead regulations. Any surfaces not tested should be considered lead containing.

This report can be reproduced only in full. This risk assessment report expires March 10, 2005.

_________________		                                                             __________________

John Doe	                                                                                     John Doe

NY-00-000000-00	                                                                         NY-00-000000-00

QUICK SUMMARY OF LEAD TESTING RESULTS

Dwelling: 123 Any Street, Syracuse NY                                                                                                                             Date: 3/10/2004

Inspector: John Doe (EPA Cert. 00-000000-00)

SAMPLE                       LOCATION                               SAMPLE TYPE                        AREA                              RESULT                         COMPILIANCE

| 001 | Room 1 floor /entrance floor | Wipe Sample | 1.00 ft2 | 271.9 µg/ft2 | n |
| --- | --- | --- | --- | --- | --- |
| 002 | Room 1 sill | Wipe Sample | 0.58 ft2 | <34.5 µg/ft2 | y |
| 003 | Kitchen floor | Wipe Sample | 1.00 ft2 | 52.7 µg/ft2 | n |
| 004 | Kitchen sill | Wipe Sample | 0.13 ft2 | <153.8 µg/ft2 | y |
| 005 | Room 4 floor | Wipe Sample | 1.00 ft2 | 22.8 µg/ft2 | y |
| 006 | Room 4 sill | Wipe Sample | 0.58 ft2 | 1,224.6 µg/ft2 | n |
| 007 | Room 6 floor | Wipe Sample | 1.00 ft2 | <20.0 µg/ft2 | y |
| 008 | Room 6 sill | Wipe Sample | 0.58 ft2 | 62.2 µg/ft2 | y |
| 009 | Field blank | Blank |  | <20.0 µg | y |

AVERAGE SETTLED DUST LEAD LEVELS OF FLOORS: 92.0 µg/ft2

AVERAGE SETTLED DUST LEVELS OF WINDOW SILLS: 369.0 µg/ft2

REFERENCE LIMITS

DUST WIPE SAMPLES 			SOIL SAMPLES				PAINT CHIP SAMPLES

Floors		 40 µg/ft2			Play/high contact areas	400 ppm 			0.5% Lead by weight

Window Sills		250 µg/ft2		Building perimeter/yard	1,200 ppm

Paving or removal criteria	5,000 ppm 		WATER SAMPLES

15 Parts per Billion (ppb)

Lead Hazard Control Visual Inspection Form

Date: 			                                                Name of Inspector:

License no:

Name of Property owner:

Property address: 								  Apt. no:

| Room ID | Condition of Components in each room | Visible paint chips? | Visible settled dust? |
| --- | --- | --- | --- |

Notes:

Signature

Field Questionnaire for Lead Risk Assessments

Location: _______________________                                                                 Date:__/__/__

1. What is the approximate age of the dwelling?

2. Where does the child like to play or hide?  Inside: ______________

Outside: _______________

3. Is the unit or are near by buildings or structures being renovated, repainted, or demolished?

4. Is there any deteriorated paint on exterior components?

5. Are there any visible paint chips near the perimeter of the house, fences, garages, or play structures?

6. Risk assessor should note any lead-related occupation or hobby.

7. Does the child have a favorite cup and/or eating utensil?        If yes, are they handing made or ceramic?

8. does the child have a pet that could track in contaminated soil or dust from the outside?

If yes, where does the pet sleep?

9. Risk assessor should note cleaning habits within dwelling.

10. Are there any detached structures on this site?

INTAKE INFORMATION

EMERGENCY CASE?__________

Case #_______________   HUD I.D.#______________	Initial Contact Date_______________________

Street #__________   Street Name________________________________________Zip_________________

No. of Units____ Owner-Occupied: Yes_____   No_____ Property I.D. Number._____ - ________________

NSA/Neighborhood_______/______ Other Property: Yes_____ No_____ Census Tract #________________

BG____ Property Tax Status: (C/D) _____   Tax Trust: Yes____ No____ Current: Yes____ No___________

Full Assessed Value $____________________

_______________________________________________________________________________________

Last Name:___________________________	Last Name:____________________________________

First Name:___________________________	First Name:____________________________________

Home Phone #________________	Work Phone #________________   Cell Phone #__________________

Contact Person:_____________________________	Phone #_________________________________

Owner’s Address if different from above:

Street #_________ Street Name______________________________________________________________

City_______________________ State____________________________ Zip Code____________________

Revised: 2/7/02 BM

Recommendations Prioritized

123 Any Street

| NO. | Component | Location | Method | Reason |
| --- | --- | --- | --- | --- |
| 1 | Windows | All windows meant to open | Remove and replace with new vinyl windows | Friction surface Poor condition |
| 2 | Window sills | Room 7 and bathroom | Wet scrape and paint | Poor condition Chewable surface |
| 3 | Ceiling | Side “A” porch | Wet scrape and paint | Poor condition |
| 4 | Header | Side “A” porch | Wet scrape and cover with aluminum coil stock | Poor condition |
| 5 | Rail cap | Side “A” porch | Wet scrape and cover with aluminum coil stock | Poor condition |
| 6 | Trim | All exterior trim | Wet scrape and cover with aluminum coil stock | Poor condition |

Because this report recommends the use of encapsulates and enclosure techniques which are not permanent solutions, it is necessary for the owner to have a monitoring and maintenance schedule. It is recommended that the owner inspect these areas for any damage or deterioration every six months and upon lease renewal. If there are any signs of deterioration, repairs should be performed by qualified personnel after which clearance testing should be performed.

Anyone that intends on doing the work will attend the one-day lead maintenance worker class for lead safe work practices.

Anything listed as fair condition should be monitored frequently by homeowner.

The notice of Lead Hazard Evaluation will be provided by the owner to the residents in the dwelling. The owner will explain to the residents that the lead hazards at the property will be corrected during renovation. After the work has been completed and clearance established, the owner will forward a notice of Lead Hazard Reduction.

Any renovation work that involves the removal or disturbance of leaded materials identified in this dwelling must be done in compliance with lead regulations. Any surfaces not tested should be considered lead containing. This report cannot be reproduced without the permission of The City of Syracuse Lead Program and then only in full.